| ETSI/SAGE Specification |  | Version: 1.1 Date: 25th October 2012 |
| --- | --- | --- |
| Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2 Document 3: Implementors’ Test Data |  |  |

| Document History |  |  |
| --- | --- | --- |
| V1.0 | 10-01-2006 | Publication |
| 1.1 | 25-10-2012 | Correction to mis-edited IV in section 5.5 |

PREFACE

This specification has been prepared by the 3GPP Task Force, and gives detailed test data for implementors of the algorithm set. It provides visibility of the internal state of the algorithm to aid in the realisation of the algorithms.

This document is the third of four, which between them form the entire specification of the 3GPP Confidentiality and Integrity Algorithms:

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 1: Algorithm Specifications.

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 2: SNOW 3G Algorithm Specification.

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 3: Implementors’ Test Data.

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 4: Design Conformance Test Data.

This document is purely informative.  The normative part of the specification of the UEA2 (confidentiality) and the UIA2 (integrity) algorithms is in the main body of document 1. The normative part of the specification of SNOW 3G is found in document 2.

Blank Page

TABLE OF CONTENTS

1.	OUTLINE OF THE IMPLEMENTORS’ TEST DATA	7

2.	INTRODUCTORY INFORMATION	7

2.1.	Introduction	7

2.2.	Radix	7

2.3.	Bit/Byte ordering	7

2.4.	Presentation of input/output data	7

3.	SNOW 3G	8

3.1.	Overview	8

3.2.	Format	8

3.3.	Test Set 1	8

3.4.	Test Set 2	9

3.5.	Test Set 3	10

3.6.	Test Set 4	11

4.	CONFIDENTIALITY ALGORITHM UEA2	12

4.1.	Overview	12

4.2.	Format	12

4.3.	Test Set 1	12

4.4.	Test Set 2	13

4.5.	Test Set 3	13

4.6.	Test Set 4	14

4.7.	Test Set 5	14

5.	INTEGRITY ALORITHM UIA2	15

5.1.	Overview	15

5.2.	Format	15

5.3.	Test Set 1	15

5.4.	Test Set 2	16

5.5.	Test Set 3	16

5.6.	Test Set 4	17

5.7.	Test Set 5	18

5.8.	Test Set 6	18

REFERENCES

- 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Security Architecture (3G TS 33.102 version 6.3.0)

- 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Cryptographic Algorithm Requirements; (3G TS 33.105 version 6.0.0)

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 1: UEA2 and UIA2 specifications.

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2. 
Document 2: SNOW 3G specification.

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 3: Implementors’ Test Data.

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 4: Design Conformance Test Data.

- P. Ekdahl and T. Johansson, “A new version of the stream cipher SNOW”, in Selected Areas in Cryptology (SAC 2002), LNCS 2595, pp. 47–61, Springer-Verlag,

# OUTLINE OF THE IMPLEMENTORS’ TEST DATA

Section 2 introduces the algorithms and describes the notation used in the subsequent sections.

Section 3 provides test data for SNOW 3G.

Section 4 provides test data for the Confidentiality Algorithm UEA2.

Section 5 provides test data for the Integrity Algorithm UIA2.

# INTRODUCTORY INFORMATION

## Introduction

Within the security architecture of the 3GPP system there are two standardised algorithms; a confidentiality algorithm UEA2, and an integrity algorithm UIA2. These algorithms are specified in a companion document [3]. Each of these algorithms is based on the SNOW 3G algorithm that is specified in [4].

To assist implementors with their realisation of the algorithm set this document provides test data for these algorithms along with extensive detail of the internal states of the algorithms as they process the given input data.

Final testing of the algorithms should be performed using the test data sets given in the “Design Conformance” companion document [6].

## Radix

Unless stated otherwise, all test data values presented in this document are in hexadecimal.

## Bit/Byte ordering

All data variables in this specification are presented with the most significant bit (or byte) on the left hand side and the least significant bit (or byte) on the right hand side. Where a variable is broken down into a number of sub-strings, the left most (most significant) sub-string is numbered 0, the next most significant is numbered 1 and so on through to the least significant.

For example the 128-kit key K is subdivided into four 32-bit substrings K0, K1, K2,, K3 so if we have a key
                         K = 0123456789ABCDEFFEDCBA9876543210

we have:

K0 = 01234567, K1 = 89ABCDEF, K2 = FEDCBA98, K3 = 76543210.

## Presentation of input/output data

The basic data processed by the UEA2 and UIA2 algorithms are bit streams. In general in this document the data is presented in hexadecimal format as bytes, thus the last byte shown as part of an input or output data stream may include between 0 and 7 bits that are ignored once the LENGTH parameter is taken into account. (The least significant bits of the byte are ignored).

# SNOW 3G

## Overview

The test data sets presented here are for the SNOW 3G stream cipher algorithm.

## Format

Each test set starts by showing the input and output data values.

This is followed by a table showing the state of the LFSR at the beginning of the computation.

Then for the first 8 steps of the initialisation the content of s0, s2, s5, s11, s15, R1, R2, R3 is given in a table.

Then the state of the LFSR and the FSM at the end of the initialisation is given.

For the first 3 steps of keystream generation s0, s2, s5, s11, s15, R1, R2, R3 are given in a table.

Finally the output z1, z2, … is given.

## Test Set 1

input:

Key:	2B D6 45 9F 82 C5 B3 00 95 2C 49 10 48 81 FF 48

IV: 	EA 02 47 14 AD 5C 4D 84 DF 1F 9B 25 1C 0B F4 5F

output:

z1: AB EE 97 04

z2: 7A C3 13 73

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| 2B D6 45 9F | 82 C5 B3 00 | 95 2C 49 10 | 48 81 FF 48 |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| EA 02 47 14 | AD 5C 4D 84 | DF 1F 9B 25 | 1C 0B F4 5F |

Initialisation Mode

LFSR-state at the beginning:

| i | S0+i | S1+i | S2+i | S3+i | S4+i | S5+i | S6+i | S7+i |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | D429BA60 | 7D3A4CFF | 6AD3B6EF | B77E00B7 | 2BD6459F | 82C5B300 | 952C4910 | 4881FF48 |
| 8 | D429BA60 | 6131B8A0 | B5CC2DCA | B77E00B7 | 868A081B | 82C5B300 | 952C4910 | A283B85C |

|  | S0 | S2 | S5 | S11 | S15 | R1 | R2 | R3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | D429BA60 | 6AD3B6EF | 82C5B300 | B77E00B7 | A283B85C | 00000000 | 00000000 | 00000000 |
| 1 | 7D3A4CFF | B77E00B7 | 952C4910 | 868A081B | 97DF2884 | 82C5B300 | 63636363 | 25252525 |
| 2 | 6AD3B6EF | 2BD6459F | 4881FF48 | 82C5B300 | 311BA301 | 136CCF98 | 486C5BC4 | 93939393 |
| 3 | B77E00B7 | 82C5B300 | D429BA60 | 952C4910 | A69FCBCB | 237EC89F | EAEBC424 | 4B7815EA |
| 4 | 2BD6459F | 952C4910 | 6131B8A0 | A283B85C | E76F0ADA | 8A3D73AE | 21A4385B | E662EC27 |
| 5 | 82C5B300 | 4881FF48 | B5CC2DCA | 97DF2884 | A52DCD12 | A8F78CE2 | 63A7F600 | BC3F3A8D |
| 6 | 952C4910 | D429BA60 | B77E00B7 | 311BA301 | 1A349A62 | 6D9B0D47 | 20712A2D | 391D0883 |
| 7 | 4881FF48 | 6131B8A0 | 868A081B | A69FCBCB | 2A2A44DB | AED43261 | 401B1511 | 45A6ED60 |

LFSR-state after completion of the initialisation mode:

| i | S0+i | S1+i | S2+i | S3+i | S4+i | S5+i | S6+i | S7+i |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 8F1215A6 | E003A052 | 9241C929 | 68D7BF8C | 16BF4C2A | 8DEF9D70 | 32381704 | 11DD346A |
| 8 | E18B81EA | 77EBD4FE | 57ED9505 | 0C33C0EF | 1A037B59 | 97591E82 | A91CCB44 | 7B48E04F |

FSM-state after completion of the initialisation mode:

R1 = 61DA9249

R2 = 427DF38C

R3 = 0FB6B101

Keystream mode

|  | S0 | S2 | S5 | S11 | S15 | R1 | R2 | R3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | E003A052 | 68D7BF8C | 32381704 | 1A037B59 | 1646644C | C4D71FFD | 90F0B31F | CC612008 |
| 1 | 9241C929 | 16BF4C2A | 11DD346A | 97591E82 | 52E43190 | 8F49EA2B | 0AACC1E1 | 3367438C |
| 2 | 68D7BF8C | 8DEF9D70 | E18B81EA | A91CCB44 | B737110E | 2D6739C7 | 5295DA23 | 5293E49E |

Output:

z1 = AB EE 97 04

z2 = 7A C3 13 73

## Test Set 2

input:

Key: 	8C E3 3E 2C C3 C0 B5 FC 1F 3D E8 A6 DC 66 B1 F3

IV: 	D3 C5 D5 92 32 7F B1 1C DE 55 19 88 CE B2 F9 B7

output:

z1: EF F8 A3 42

z2: F7 51 48 0F

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| 8C E3 3E 2C | C3 C0 B5 FC | 1F 3D E8 A6 | DC 66 B1 F3 |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| D3 C5 D5 92 | 32 7F B1 1C | DE 55 19 88 | CE B2 F9 B7 |

Initialisation Mode

LFSR-state at the beginning:

| i | S0+i | S1+i | S2+i | S3+i | S4+i | S5+i | S6+i | S7+i |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 731CC1D3 | 3C3F4A03 | E0C21759 | 23994E0C | 8CE33E2C | C3C0B5FC | 1F3DE8A6 | DC66B1F3 |
| 8 | 731CC1D3 | F28DB3B4 | 3E970ED1 | 23994E0C | BE9C8F30 | C3C0B5FC | 1F3DE8A6 | 0FA36461 |

|  | S0 | S2 | S5 | S11 | S15 | R1 | R2 | R3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 731CC1D3 | E0C21759 | C3C0B5FC | 23994E0C | 0FA36461 | 00000000 | 00000000 | 00000000 |
| 1 | 3C3F4A03 | 23994E0C | 1F3DE8A6 | BE9C8F30 | EF81E474 | C3C0B5FC | 63636363 | 25252525 |
| 2 | E0C21759 | 8CE33E2C | DC66B1F3 | C3C0B5FC | 7A554815 | 9D7C30E6 | F878FA8B | 93939393 |
| 3 | 23994E0C | C3C0B5FC | 731CC1D3 | 1F3DE8A6 | 53E0AE66 | 486E1CEB | 2148E845 | 098F198B |
| 4 | 8CE33E2C | 1F3DE8A6 | F28DB3B4 | 0FA36461 | 9A1EE9B8 | 9BDCC09D | 87A622BB | EFFA4239 |
| 5 | C3C0B5FC | DC66B1F3 | 3E970ED1 | EF81E474 | 2390FE04 | A51E1448 | F6CFB4FB | 2087DC1D |
| 6 | 1F3DE8A6 | 731CC1D3 | 23994E0C | 7A554815 | 6FB8C36C | 14E087C7 | 72462DC5 | 0B8BF471 |
| 7 | DC66B1F3 | F28DB3B4 | BE9C8F30 | 53E0AE66 | BA5DB98F | 9A58E842 | 481D2AB5 | 5C8EE565 |

LFSR-state after completion of the initialisation mode:

| i | S0+i | S1+i | S2+i | S3+i | S4+i | S5+i | S6+i | S7+i |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 04D6A929 | 942E1440 | 82ABD3FE | 5832E9F4 | 5F9702A0 | 08712C81 | 644CC9B9 | DBF6DE13 |
| 8 | BAA5B1D0 | 92E9DD53 | A2E2FA6D | CE6965AA | 02C0CD4E | 6E6D984F | 114A90E7 | 5279F8DA |

FSM-state after completion of the initialisation mode:

R1 = 65130120

R2 = A14C7DBD

R3 = B68B551A

Keystream mode

|  | S0 | S2 | S5 | S11 | S15 | R1 | R2 | R3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 942E1440 | 5832E9F4 | 644CC9B9 | 02C0CD4E | C1E93B6B | 6046F758 | 59E685C1 | 7DCBC989 |
| 1 | 82ABD3FE | 5F9702A0 | DBF6DE13 | 6E6D984F | CEB99926 | 736D85F1 | 37DD84E6 | A9BECBB1 |
| 2 | 5832E9F4 | 08712C81 | BAA5B1D0 | 114A90E7 | E34F6919 | AA259A88 | 56C45F48 | C3546A61 |

Output:

z1 = EF F8 A3 42

z2 = F7 51 48 0F

## Test Set 3

input:

Key:	40 35 C6 68 0A F8 C6 D1 A8 FF 86 67 B1 71 40 13

IV: 	62 A5 40 98 1B A6 F9 B7 45 92 B0 E7 86 90 F7 1B

output:

z1: A8 C8 74 A9

z2: 7A E7 C4 F8

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| 40 35 C6 68 | 0A F8 C6 D1 | A8 FF 86 67 | B1 71 40 13 |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| 62 A5 40 98 | 1B A6 F9 B7 | 45 92 B0 E7 | 86 90 F7 1B |

Initialisation Mode

LFSR-state at the beginning:

| i | S0+i | S1+i | S2+i | S3+i | S4+i | S5+i | S6+i | S7+i |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | BFCA3997 | F507392E | 57007998 | 4E8EBFEC | 4035C668 | 0AF8C6D1 | A8FF8667 | B1714013 |
| 8 | BFCA3997 | 7397CE35 | 1292C97F | 4E8EBFEC | 5B933FDF | 0AF8C6D1 | A8FF8667 | D3D4008B |

|  | S0 | S2 | S5 | S11 | S15 | R1 | R2 | R3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | BFCA3997 | 57007998 | 0AF8C6D1 | 4E8EBFEC | D3D4008B | 00000000 | 00000000 | 00000000 |
| 1 | F507392E | 4E8EBFEC | A8FF8667 | 5B933FDF | EE2CABF5 | 0AF8C6D1 | 63636363 | 25252525 |
| 2 | 57007998 | 4035C668 | B1714013 | 0AF8C6D1 | 667356A3 | F13E06A5 | 79A1E99D | 93939393 |
| 3 | 4E8EBFEC | 0AF8C6D1 | BFCA3997 | A8FF8667 | 6410181D | 9C84BD1D | 8EEEB4AE | E5995CC4 |
| 4 | 4035C668 | A8FF8667 | 7397CE35 | D3D4008B | 241A7790 | E9421A01 | 75196F5C | C83E1776 |
| 5 | 0AF8C6D1 | B1714013 | 1292C97F | EE2CABF5 | C485B826 | 30C3489F | 36A44937 | 0F317420 |
| 6 | A8FF8667 | BFCA3997 | 4E8EBFEC | 667356A3 | A211C1E9 | 54480696 | 02D90971 | 3D982023 |
| 7 | B1714013 | 7397CE35 | 5B933FDF | 6410181D | 6E8AE7E6 | 75EFA940 | D63B98F8 | 883F13A7 |

LFSR-state after completion of the initialisation mode:

| i | S0+i | S1+i | S2+i | S3+i | S4+i | S5+i | S6+i | S7+i |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | FEAFBAD8 | 1B11050A | 23708014 | AC8494DB | ED97D431 | DBBB59B3 | 6CD30005 | 7EC36405 |
| 8 | B20F02AC | EB407735 | 50E41A0E | FFA8ABC1 | EB4800A7 | D4E6749D | D1C452FE | A92A3153 |

FSM-state after completion of the initialisation mode:

R1 = 6599AA50

R2 = 5EA9188B

R3 = F41889FC

Keystream mode

|  | S0 | S2 | S5 | S11 | S15 | R1 | R2 | R3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1B11050A | AC8494DB | 6CD30005 | EB4800A7 | 0FE91C6F | 8E4CE8DA | 2DEF74EA | 42B4B0A3 |
| 1 | 23708014 | ED97D431 | 7EC36405 | D4E6749D | C3CB3734 | 5C572590 | 79B51828 | 2496A1E1 |
| 2 | AC8494DB | DBBB59B3 | B20F02AC | D1C452FE | 739AB29C | D40ADE0C | 5037B990 | 32D1FAE0 |

Output:

z1 = A8 C8 74 A9

z2 = 7A E7 C4 F8

## Test Set 4

This test ensures that all entries in the tables S_R, T0, T1, T2, T3, S2_T0, S2_T1, S2_T2, S2_T3 and MUL, DIV are correct. For a fixed key and IV the algorithm is clocked 2500 times in keystream mode. With the given data every entry will be used at least once.

Iterated test for full tables coverage

input:

Key:	0D ED 72 63 10 9C F9 2E 33 52 25 5A 14 0E 0F 76

IV: 	6B 68 07 9A 41 A7 C4 C9 1B EF D7 9F 7F DC C2 33

output:

z1:	D7 12 C0 5C

z2:	A9 37 C2 A6

z3:	EB 7E AA E3

…

z2500:	9C 0D B3 AA

# CONFIDENTIALITY ALGORITHM UEA2

## Overview

The test data sets presented here are for the UEA2 confidentiality algorithm. No detailed data is presented for the internal states of SNOW 3G as that is covered in section 3.

## Format

Each test set starts by showing the various inputs to the algorithm including the data stream to be encrypted/decrypted.  (The length field is in decimal).  This is followed by:

the key words K0, K1, K2, K3

the Initialisation Variables IV0, IV1, IV2, IV3.

Thereafter three columns of data are shown.

Word number	shows the number of the current 32-bit word.

Keystream	shows the 32-bit output from SNOW 3G.

Enc/dec data	shows the modified input data, i.e. it is the bitwise exclusive-or of the corresponding keystream and the input data to the algorithm.  As this is a bitwise stream cipher it is purely a matter of context whether the operation is regarded as “encryption” or “decryption”.

## Test Set 1

Count-C		= 72A4F20F

Bearer		= 0C

Direction	= 1

CK			= 2B D6 45 9F 82 C5 B3 00 95 2C 49 10 48 81 FF 48

Length		= 798 bits

Plaintext:

7EC61272 743BF161 4726446A 6C38CED1

66F6CA76 EB543004 4286346C EF130F92

922B0345 0D3A9975 E5BD2EA0 EB55AD8E

1B199E3E C4316020 E9A1B285 E7627953

59B7BDFD 39BEF4B2 484583D5 AFE082AE

E638BF5F D5A60619 3901A08F 4AB41AAB

9B134880

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| 48 81 FF 48 | 95 2C 49 10 | 82 C5 B3 00 | 2B D6 45 9F |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| 64 00 00 00 | 72 A4 F2 0F | 64 00 00 00 | 72 A4 F2 0F |

| Wordnumber | Keystream | enc/dec data |
| --- | --- | --- |
| 0 | F22DB45B 37E71C5B | 8CEBA629 43DCED3A |
| 2 | 4EB6F404 CD886C15 | 0990B06E A1B0A2C4 |
| 4 | 9DCA27B1 F062AF46 | FB3CEDC7 1B369F42 |
| 6 | F8E2F587 8976E8B8 | BA64C1EB 6665E72A |
| 8 | 33E2B848 E798969D | A1C9BB0D EAA20FE8 |
| 10 | 85E5961A 057983F1 | 6058B8BA EE2C2E7F |
| 12 | 10F55076 71185285 | 0BECCE48 B52932A5 |
| 14 | D53CED16 FD580500 | 3C9D5F93 1A3A7C53 |
| 16 | 7BEE12BE 1C5C52EC | 2259AF43 25E2A65E |
| 18 | 78C12E8A C5B1B9D5 | 3084AD5F 6A513B7B |
| 20 | 3BF90900 DF06DF63 | DDC1B65F 0AA0D97A |
| 22 | 3C3C15D5 C270DE52 | 053DB55A 88C4C4F9 |
| 24 | FB4D09C0 | 605E4140 |

## Test Set 2

Count-C		= E28BCF7B

Bearer		= 18

Direction	= 0

CK			= EF A8 B2 22 9E 72 0C 2A 7C 36 EA 55 E9 60 56 95

Length		= 510 bits

Plaintext:

10111231 E060253A 43FD3F57 E37607AB

2827B599 B6B1BBDA 37A8ABCC 5A8C550D

1BFB2F49 4624FB50 367FA36C E3BC68F1

1CF93B15 10376B02 130F812A 9FA169D8

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| E9 60 56 95 | 7C 36 EA 55 | 9E 72 0C 2A | EF A8 B2 22 |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| C0 00 00 00 | E2 8B CF 7B | C0 00 00 00 | E2 8B CF 7B |

| Wordnumber | Keystream | enc/dec data |
| --- | --- | --- |
| 0 | F0CB07FB 6E4571CF | E0DA15CA 8E2554F5 |
| 2 | A691AB3F 3F1A7BB9 | E56C9468 DC6C7C12 |
| 4 | B4713F3C B592AC3A | 9C568AA5 032317E0 |
| 6 | 79AF82A8 3627BAAB | 4E072964 6CABEFA6 |
| 8 | 927D6308 49000249 | 89864C41 0F24F919 |
| 10 | D0619E91 196B16A7 | E61E3DFD FAD77E56 |
| 12 | 114992D8 26F421E6 | 0DB0A9CD 36C34AE4 |
| 14 | 0B1B1198 00FECB24 | 181490B2 9F5FA2FC |

## Test Set 3

Count-C		= FA556B26

Bearer		= 03

Direction	= 1

CK			= 5A CB 1D 64 4C 0D 51 20 4E A5 F1 45 10 10 D8 52

Length		= 120 bits

Plaintext:

AD9C441F 890B38C4 57A49D42 1407E8

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| 10 10 D8 52 | 4E A5 F1 45 | 4C 0D 51 20 | 5A CB 1D 64 |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| 1C 00 00 00 | FA 55 6B 26 | 1C 00 00 00 | FA 55 6B 26 |

| Wordnumber | Keystream | enc/dec data |
| --- | --- | --- |
| 0 | 1793752F 8A3FFDAF | BA0F3130 0334C56B |
| 2 | 0503D43E AEC7AE | 52A7497C BAC046 |

## Test Set 4

Count-C		= 398A59B4

Bearer		= 05

Direction	= 1

CK			= D3 C5 D5 92 32 7F B1 1C 40 35 C6 68 0A F8 C6 D1

Length		= 253 bits

Plaintext:

981BA682 4C1BFB1A B4854720 29B71D80

8CE33E2C C3C0B5FC 1F3DE8A6 DC66B1F0

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| 0A F8 C6 D1 | 40 35 C6 68 | 32 7F B1 1C | D3 C5 D5 92 |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| 2C 00 00 00 | 39 8A 59 B4 | 2C 00 00 00 | 39 8A 59 B4 |

| Wordnumber | Keystream | enc/dec data |
| --- | --- | --- |
| 0 | 0080D71E 902835AD | 989B719C DC33CEB7 |
| 2 | 7BA22D72 ABCBF214 | CF276A52 827CEF94 |
| 4 | 298F7EEC 685D340B | A56C40C0 AB9D81F7 |
| 6 | BD945260 D2777540 | A2A9BAC6 0E11C4B0 |

## Test Set 5

Count-C		= 72A4F20F

Bearer		= 09

Direction	= 0

CK			= 60 90 EA E0 4C 83 70 6E EC BF 65 2B E8 E3 65 66

Length		= 837 bits

Plaintext:

40981BA6 824C1BFB 4286B299 783DAF44

2C099F7A B0F58D5C 8E46B104 F08F01B4

1AB48547 2029B71D 36BD1A3D 90DC3A41

B46D5167 2AC4C966 3A2BE063 DA4BC8D2

808CE33E 2CCCBFC6 34E1B259  60876A0

FBB5A437 EBCC8D31 C19E4454 318745E3

98764598 7A986F2C B0

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| E8 E3 65 66 | EC BF 65 2B | 4C 83 70 6E | 60 90 EA E0 |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| 48 00 00 00 | 72 A4 F2 0F | 48 00 00 00 | 72 A4 F2 0F |

| Wordnumber | Keystream | enc/dec data |
| --- | --- | --- |
| 0 | 180AA00E 09F7D155 | 5892BBA8 8BBBCAAE |
| 2 | ECF02839 1355927E | AE769AA0 6B683D3A |
| 4 | 3BC59BD9 D97D9BCB | 17CC04A3 69881697 |
| 6 | CD18F5FA 25709B41 | 435E44FE D5FF9AF5 |
| 8 | 612A0C4A 6D75D36D | 7B9E890D 4D5C6470 |
| 10 | AE38CEB7 74DAAAAD | 9885D48A E40690EC |
| 12 | B056FB8E 5A935F82 | 043BAAE9 705796E4 |
| 14 | 93D4BA28 57C0FE05 | A9FF5A4B 8D8B36D7 |
| 16 | 7372B4F2 4031D316 | F3FE57CC 6CFD6CD0 |
| 18 | 312C8A0B AE56E26E | 05CD3852 A85E94CE |
| 20 | 907834E7 3BB4B4FF | 6BCD90D0 D07839CE |
| 22 | C8ED7110 FB0970EB | 09733544 CA8E3508 |
| 24 | DB52C0C8 E8B2AE04 | 43248550 922AC128 |
| 26 | A8 | 18 |

# INTEGRITY ALORITHM UIA2

## Overview

The test data sets presented here are for the UIA2 integrity algorithm. No detailed data is presented for the internal states of SNOW 3G as that is covered in section 3.

## Format

The test data set shows the input values to the algorithm.

This is followed by:

the key words K0, K1, K2, K3

the Initialisation Variables IV0, IV1, IV2 and IV3

the keystream words z1, z2, z3,, z4, z5.

the value P = z1 || z2

the value Q = z3 || z4.

Then for each message word Mi, 0  i  D-1 this word Mi and the intermediate value EVAL are given. After that the result of the multiplication of EVAL by Q is displayed.

Finally the output MAC-I of the UIA2-algorithm is shown.

## Test Set 1

COUNT-I    	= 38A6F056

FRESH      	= 05D2EC49

DIRECTION 	= 0

IK       	= 2B D6 45 9F 82 C5 B3 00 95 2C 49 10 48 81 FF 48

LENGTH   	= 189 bits

MESSAGE:

6B227737296F393C 8079353EDC87E2E8 05D2EC49A4F2D8E0

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| 48 81 FF 48 | 95 2C 49 10 | 82 C5 B3 00 | 2B D6 45 9F |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| 05 D2 EC 49 | 38 A6 F0 56 | 05 D2 EC 49 | 38 A6 F0 56 |

| z1 | z2 | z3 | z4 | z5 |
| --- | --- | --- | --- | --- |
| DC 0D 53 25 | 2A 5D 31 90 | 7E 1B 8E 28 | 25 EC 4C AA | 63 D9 C7 7C |

P= DC 0D 53 25  2A 5D 31 90

Q= 7E 1B 8E 28  25 EC 4C AA

i           Mi                   EVAL

0 	 6B227737 296F393C 	  8BA78DCD 0D8C242D

1 	 8079353E DC87E2E8 	  7559CCE4 3F4DCEB5

2 	 05D2EC49 A4F2D8E0 	  8C108081 F386B04E

3 	 00000000 000000BD 	  8C108081 F386B0F3

Multiply by Q: EVAL= 4817DF5C 251B5E20

MAC-I: 2BCE1820

## Test Set 2

COUNT-I    	= 3EDC87E2

FRESH      	= A4F2D8E2

DIRECTION 	= 1

IK       	= D4 2F 68 24 28 20 1C AF CD 9F 97 94 5E 6D E7 B7

LENGTH   	= 254 bits

MESSAGE:

B5924384328A4AE0 0B737109F8B6C8DD 2B4DB63DD533981C EB19AAD52A5B2BC0

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| 5E 6D E7 B7 | CD 9F 97 94 | 28 20 1C AF | D4 2F 68 24 |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| A4 F2 58 E2 | BE DC 87 E2 | A4 F2 D8 E2 | 3E DC 87 E2 |

| z1 | z2 | z3 | z4 | z5 |
| --- | --- | --- | --- | --- |
| 67 0E 29 DE | 2A D6 DE 7E | A4 2A D0 48 | 40 7A 24 AC | 20 F8 60 70 |

P= 67 0E 29 DE  2A D6 DE 7E

Q= A4 2A D0 48  40 7A 24 AC

i           Mi                   EVAL

0 	 B5924384 328A4AE0 	  E7354091 E1B57157

1 	 0B737109 F8B6C8DD 	  655CA81A A179F483

2 	 2B4DB63D D533981C 	  E6E0FD58 B1B4BA89

3 	 EB19AAD5 2A5B2BC0 	  9BC353AA 5FE30866

4 	 00000000 000000FE 	  9BC353AA 5FE30898

Multiply by Q: EVAL= DC8378CD FD41FE17

MAC-I: FC7B18BD

## Test Set 3

COUNT-I    	= 36AF6144

FRESH      	= 9838F03A

DIRECTION 	= 1

IK       	= FD B9 CF DF 28 93 6C C4 83 A3 18 69 D8 1B 8F AB

LENGTH   	= 319 bits

MESSAGE:

5932BC0ACE2B0ABA 33D8AC188AC54F34 6FAD10BF9DEE2920 B43BD0C53A915CB7

DF6CAA72053ABFF2

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| D8 1B 8F AB | 83 A3 18 69 | 28 93 6C C4 | FD B9 CF DF |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| 98 38 70 3A | B6 AF 61 44 | 98 38 F0 3A | 36 AF 61 44 |

| z1 | z2 | z3 | z4 | z5 |
| --- | --- | --- | --- | --- |
| B3 9A FB 5D | 53 AA 27 D4 | 56 A1 C4 AE | CB 68 F9 1A | BF 27 34 7B |

P= B3 9A FB 5D  53 AA 27 D4

Q= 56 A1 C4 AE  CB 68 F9 1A

i           Mi                   EVAL

0 	 5932BC0A CE2B0ABA 	  6E988791 F4F8ADD7

1 	 33D8AC18 8AC54F34 	  39723954 579492CB

2 	 6FAD10BF 9DEE2920 	  EEEAC385 C4D5E0C0

3 	 B43BD0C5 3A915CB7 	  EB79B071 CBAECF56

4 	 DF6CAA72 053ABFF2 	  32114B23 317FA002

5 	 00000000 0000013F 	  32114B23 317FA13D

Multiply by Q: EVAL= BDD6CED4 C458544C

MAC-I: 02F1FAAF

## Test Set 4

COUNT-I    	= 14793E41

FRESH      	= 0397E8FD

DIRECTION 	= 1

IK       	= C7 36 C6 AA B2 2B FF F9 1E 26 98 D2 E2 2A D5 7E

LENGTH   	= 384 bits

MESSAGE:

D0A7D463DF9FB2B2 78833FA02E235AA1 72BD970C1473E129 07FB648B6599AAA0

B24A038665422B20 A499276A50427009

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| E2 2A D5 7E | 1E 26 98 D2 | B2 2B FF F9 | C7 36 C6 AA |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| 03 97 68 FD | 94 79 3E 41 | 03 97 E8 FD | 14 79 3E 41 |

| z1 | z2 | z3 | z4 | z5 |
| --- | --- | --- | --- | --- |
| 45 89 8E 82 | 8F 27 EB 98 | E3 23 07 09 | A0 0C B7 0A | 8F 75 AC 4B |

P= 45 89 8E 82  8F 27 EB 98

Q= E3 23 07 09  A0 0C B7 0A

i           Mi                   EVAL

0 	 D0A7D463 DF9FB2B2 	  9E80B47B 98010914

1 	 78833FA0 2E235AA1 	  5EA34890 532D5FFB

2 	 72BD970C 1473E129 	  0EAE8E55 95661FCF

3 	 07FB648B 6599AAA0 	  7EA00D6D 65C8F93F

4 	 B24A0386 65422B20 	  BEC91666 B07F7551

5 	 A499276A 50427009 	  689EF151 53554DC2

6 	 00000000 00000180 	  689EF151 53554C42

Multiply by Q: EVAL= B7C0F88B 24B5417C

MAC-I: 38B554C0

## Test Set 5

COUNT-I    	= 296F393C

FRESH      	= 6B227737

DIRECTION 	= 1

IK       	= F4 EB EC 69 E7 3E AF 2E B2 CF 6A F4 B3 12 0F FD

LENGTH   	= 1000 bits

MESSAGE:

10BFFF839E0C7165 8DBB2D1707E14572 4F41C16F48BF403C 3B18E38FD5D1663B

6F6D900193E3CEA8 BB4F1B4F5BE82203 2232A78D7D75238D 5E6DAECD3B4322CF

59BC7EA84AB18811 B5BFB7BC553F4FE4 4478CE287A148799 90D18D12CA79D2C8

55149021CD5CE8CA 0371CA04FCCE143E 3D7CFEE94585B588 5CAC46068B

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| B3 12 0F FD | B2 CF 6A F4 | E7 3E AF 2E | F4 EB EC 69 |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| 6B 22 F7 37 | A9 6F 39 3C | 6B 22 77 37 | 29 6F 39 3C |

| z1 | z2 | z3 | z4 | z5 |
| --- | --- | --- | --- | --- |
| 99 14 88 47 | 1C 79 03 08 | 66 2D 90 AA | FA C5 92 D2 | 05 8B EA 75 |

P= 99 14 88 47  1C 79 03 08

Q= 66 2D 90 AA  FA C5 92 D2

i           Mi                   EVAL

0 	 10BFFF83 9E0C7165 	  012195E9 7A42A6A9

1 	 8DBB2D17 07E14572 	  AA21E590 9BF9218F

2 	 4F41C16F 48BF403C 	  9102FF2C FA4C4906

3 	 3B18E38F D5D1663B 	  5243F583 1D672845

4 	 6F6D9001 93E3CEA8 	  18BC08D1 186CA669

5 	 BB4F1B4F 5BE82203 	  31D2F689 B033849E

6 	 2232A78D 7D75238D 	  33E9FCFC 7BFA4A8E

7 	 5E6DAECD 3B4322CF 	  0305A650 808ECF4E

8 	 59BC7EA8 4AB18811 	  923C4E45 E2F6BD66

9 	 B5BFB7BC 553F4FE4 	  4DEE3814 18B4C03B

10 	 4478CE28 7A148799 	  705DF239 099FB08B

11 	 90D18D12 CA79D2C8 	  AD00C27B 09065FA0

12 	 55149021 CD5CE8CA 	  52079A4B 3518C204

13 	 0371CA04 FCCE143E 	  392E7593 A8C1E40F

14 	 3D7CFEE9 4585B588 	  692A55BE F50F6B7F

15 	 5CAC4606 8B000000 	  1D034F2B EAADE93F

16 	 00000000 000003E8 	  1D034F2B EAADEAD7

Multiply by Q: EVAL= 039CAFDB C799E383

MAC-I: 061745AE

## Test Set 6

This test ensures that all entries in the tables PM0, PM1, …, PM7 are correct. The message is chosen such that every entry of every table PM0, PM1, …, PM7 is used once.

COUNT-I    	= 296F393C

FRESH      	= 6B227737

DIRECTION 	= 1

IK       	= B3 12 0F FD B2 CF 6A F4 E7 3E AF 2E F4 EB EC 69

LENGTH   	= 16448 bits

MESSAGE:

0000000000000000 0101010101010101 E0958045F3A0BBA4 E3968346F0A3B8A7

C02A018AE6407652 26B987C913E6CBF0 83570016CF83EFBC 61C082513E21561A

427C009D28C298EF ACE78ED6D56C2D45 05AD032E9C04DC60 E73A81696DA665C6

C48603A57B45AB33 221585E68EE31691 87FB0239528632DD 656C807EA3248B7B

46D002B2B5C7458E B85B9CE95879E034 0859055E3B0ABBC3 EACE8719CAA80265

C97205D5DC4BCC90 2FE1839629ED7132 8A0F0449F588557E 6898860E042AECD8

4B2404C212C9222D A5BF8A89EF679787 0CF50771A60F66A2 EE62853657ADDF04

CDDE07FA414E11F1 2B4D81B9B4E8AC53 8EA30666688D881F 6C348421992F31B9

4F8806ED8FCCFF4C 9123B89642527AD6 13B109BF75167485 F1268BF884B4CD23

D29A0934925703D6 34098F7767F1BE74 91E708A8BB949A38 73708AEF4A36239E

50CC08235CD5ED6B BE578668A17B58C1 171D0B90E813A9E4 F58A89D719B11042

D6360B1B0F52DEB7 30A58D58FAF46315 954B0A8726914759 77DC88C0D733FEFF

54600A0CC1D0300A AAEB94572C6E95B0 1AE90DE04F1DCE47 F87E8FA7BEBF77E1

DBC20D6BA85CB914 3D518B285DFA04B6 98BF0CF7819F20FA 7A288EB0703D995C

59940C7C66DE57A9 B70F82379B70E203 1E450FCFD2181326 FCD28D8823BAAA80

DF6E0F4435596475 39FD8907C0FFD9D7 9C130ED81C9AFD9B 7E848C9FED38443D

5D380E53FBDB8AC8 C3D3F06876054F12 2461107DE92FEA09 C6F6923A188D53AF

E54A10F60E6E9D5A 03D996B5FBC820F8 A637116A27AD04B4 44A0932DD60FBD12

671C11E1C0EC73E7 89879FAA3D42C64D 20CD1252742A3768 C25A901585888ECE

E1E612D9936B403B 0775949A66CDFD99 A29B1345BAA8D9D5 400C91024B0A6073

63B013CE5DE9AE86 9D3B8D95B0570B3C 2D391422D32450CB CFAE96652286E96D

EC1214A934652798 0A8192EAC1C39A3A AF6F15351DA6BE76 4DF89772EC0407D0

6E4415BEFAE7C925 80DF9BF507497C8F 2995160D4E218DAA CB02944ABF83340C

E8BE1686A960FAF9 0E2D90C55CC6475B ABC3171A80A36317 4954955D7101DAB1

6AE8179167E21444 B443A9EAAA7C91DE 36D118C39D389F8D D4469A846C9A262B

F7FA18487A79E8DE 11699E0B8FDF557C B48719D453BA7130 56109B93A218C896

75AC195FB4FB0663 9B3797144955B3C9 327D1AEC003D42EC D0EA98ABF19FFB4A

F3561A67E77C35BF 15C59C2412DA881D B02B1BFBCEBFAC51 52BC99BC3F1D15F7

71001B7029FEDB02 8F8B852BC4407EB8 3F891C9CA733254F DD1E9EDB56919CE9

FEA21C174072521C 18319A54B5D4EFBE BDDF1D8B69B1CBF2 5F489FCC98137254

7CF41D008EF0BCA1 926F934B735E090B 3B251EB33A36F82E D9B29CF4CB944188

FA0E1E38DD778F7D 1C9D987B28D132DF B9731FA4F4B41693 5BE49DE30516AF35

78581F2F13F561C0 663361941EAB249A 4BC123F8D15CD711 A956A1BF20FE6EB7

8AEA2373361DA042 6C79A530C3BB1DE0 C99722EF1FDE39AC 2B00A0A8EE7C800A

08BC2264F89F4EFF E627AC2F0531FB55 4F6D21D74C590A70 ADFAA390BDFBB3D6

8E46215CAB187D23 68D5A71F5EBEC081 CD3B20C082DBE4CD 2FACA28773795D6B

0C10204B659A939E F29BBE1088243624 429927A7EB576DD3 A00EA5E01AF5D475

83B2272C0C161A80 6521A16FF9B0A722 C0CF26B025D5836E 2258A4F7D4773AC8

01E4263BC294F43D EF7FA8703F3A4197 463525887652B0B2 A4A2A7CF87F00914

871E25039113C7E1 618DA34064B57A43 C463249FB8D05E0F 26F4A6D84972E7A9

054824145F91295C DBE39A6F920FACC6 59712B46A54BA295 BBE6A90154E91B33

985A2BCD420AD5C6 7EC9AD8EB7AC6864 DB272A516BC94C28 39B0A8169A6BF58E

1A0C2ADA8C883B7B F497A49171268ED1 5DDD2969384E7FF4 BF4AAB2EC9ECC652

9CF629E2DF0F08A7 7A65AFA12AA9B505 DF8B287EF6CC9149 3D1CAA39076E28EF

1EA028F5118DE61A E02BB6AEFC3343A0 50292F199F401857 B2BEAD5E6EE2A1F1

91022F9278016F04 7791A9D18DA7D2A6 D27F2E0E51C2F6EA 30E8AC49A0604F4C

13542E85B68381B9 FDCFA0CE4B2D3413 54852D360245C536 B612AF71F3E77C90

95AE2DBDE504B265 733DABFE10A20FC7 D6D32C21CCC72B8B 3444AE663D65922D

17F82CAA2B865CD8 8913D291A6589902 6EA1328439723C19 8C36B0C3C8D085BF

AF8A320FDE334B4A 4919B44C2B95F6E8 ECF73393F7F0D2A4 0E60B1D406526B02

2DDC331810B1A5F7 C347BD53ED1F105D 6A0D30ABA477E178 889AB2EC55D558DE

AB2630204336962B 4DB5B663B6902B89 E85B31BC6AF50FC5 0ACCB3FB9B57B663

297031378DB47896 D7FBAF6C600ADD2C 67F936DB037986DB 856EB49CF2DB3F7D

A6D23650E438F188 4041B013119E4C2A E5AF37CCCDFB6866 0738B58B3C59D1C0

248437472ABA1F35 CA1FB90CD714AA9F 635534F49E7C5BBA 81C2B6B36FDEE21C

A27E347F793D2CE9 44EDB23C8C9B914B E10335E350FEB507 0394B7A4A15C0CA1

20283568B7BFC254 FE838B137A2147CE 7C113A3A4D65499D 9E86B87DBCC7F03B

BD3A3AB1AA243ECE 5BA9BCF25F82836C FE473B2D83E7A720 1CD0B96A72451E86

3F6C3BA664A6D073 D1F7B5ED990865D9 78BD3815D06094FC 9A2ABA5221C22D5A

B996389E3721E3AF 5F05BEDDC2875E0D FAEB39021EE27A41 187CBB45EF40C3E7

3BC03989F9A30D12 C54BA7D2141DA8A8 75493E65776EF35F 97DEBC2286CC4AF9

B4623EEE902F840C 52F1B8AD658939AE F71F3F72B9EC1DE2 1588BD35484EA444

36343FF95EAD6AB1 D8AFB1B2A303DF1B 71E53C4AEA6B2E3E 9372BE0D1BC99798

B0CE3CC10D2A596D 565DBA82F88CE4CF F3B33D5D24E9C083 1124BF1AD54B7925

32983DD6C3A8B7D0

| K0 | K1 | K2 | K3 |
| --- | --- | --- | --- |
| F4 EB EC 69 | E7 3E AF 2E | B2 CF 6A F4 | B3 12 0F FD |

| IV0 | IV1 | IV2 | IV3 |
| --- | --- | --- | --- |
| 6B 22 F7 37 | A9 6F 39 3C | 6B 22 77 37 | 29 6F 39 3C |

| z1 | z2 | z3 | z4 | z5 |
| --- | --- | --- | --- | --- |
| EC 81 B3 C2 | 3C CF 81 87 | 61 F7 63 FF | 4B A3 D3 7A | 12 C6 F4 AC |

P= EC 81 B3 C2 3C CF 81 87

Q= 61 F7 63 FF 4B A3 D3 7A

i           Mi             EVAL

0 	 00000000 00000000 	  00000000 00000000

1 	 01010101 01010101 	  E1948144 F2A1BAA5

2 	 E0958045 F3A0BBA4 	  E1948144 F2A1BAA5

3 	 E3968346 F0A3B8A7 	  C3290289 E5437551

4 	 C02A018A E6407652 	  22BD83CD 17E2CFF4

…

255 	 1124BF1A D54B7925 	  CD67C229 3C57482F

256 	 32983DD6 C3A8B7D0 	  2CF3436D CEF6F28A

257 	 00000000 00004040 	  2CF3436D CEF6B2CA

Multiply by Q: EVAL= 0559DB0A B7D8E5A3

MAC-I: 179F2FA6

<End of Document>