| ETSI/SAGE Specification |  | Version: 1.1 Date: 6th September 2006 |
| --- | --- | --- |
| Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2. Document 2: SNOW 3G Specification |  |  |

The SNOW 3G algorithm is the core of the standardised 3GPP Confidentiality and Integrity algorithms UEA2 & UIA2.

| Document History |  |  |
| --- | --- | --- |
| V1.0 | 10th January 2006 | Publication |
| V1.1 | 6th September 2006 | No change to the algorithm specification at all, just removal of an unwanted page header |

PREFACE

This specification has been prepared by the 3GPP Task Force, and gives a detailed specification of the 3GPP Algorithm SNOW 3G. SNOW 3G is a stream cipher that forms the heart of the 3GPP confidentiality algorithm UEA2 and the 3GPP integrity algorithm UIA2.

This document is the second of four, which between them form the entire specification of 3GPP Confidentiality and Integrity Algorithms:

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 1: UEA2 and UIA2 Algorithm Specifications.

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2. 
Document 2: SNOW 3G Algorithm Specification.

- Specification of the 3GPP Encryption and Confidentiality Algorithms UEA2 & UIA2.
Document 3: Implementors’ Test Data.

- Specification of the 3GPP Encryption and Confidentiality Algorithms UEA2 & UIA2.
Document 4: Design Conformance Test Data.

The normative part of the specification of SNOW 3G is in the main body of this document. The annexes to this document are purely informative. Annex 1 contains remarks about the mathematical background of some functions of SNOW 3G. Annex 2 contains implementation options for some functions of SNOW 3G. Annex 3 contains illustrations of functional elements of the algorithm, while Annex 4 contains an implementation program listing of the cryptographic algorithm specified in the main body of this document, written in the programming language C.

Similarly the normative part of the specification of the UEA2 (confidentiality) and the UIA2 (integrity) algorithms is in the main body of Document 1. The annexes of those documents and Documents 3 and 4 above, are purely informative.

Blank Page

TABLE OF CONTENTS

1.	Outline of the Normative Part of the Document	8

2.	Introductory Information	8

2.1.	Introduction	8

2.2.	Notation	8

3.	Components of SNOW 3G	10

3.1.	Functions used in different Components of SNOW 3G	10

3.2.	Linear Feedback Shift Register (LFSR)	10

3.3.	Finite State Machine (FSM)	10

3.4.	The Clocking Operations	11

4.	Operation of SNOW 3G	12

4.1.	Initialisation	12

4.2.	Generation of Keystream	13

5.	Definition of Tables used in SNOW 3G	14

ANNEX 1	Remarks about the mathematical background of some operations of the SNOW 3G Algorithm	17

1.1	MULx and MULxPOW	17

1.2	The S-Box S1 used in the FSM	17

1.3	The S-Box SQ used in the S-Box S2	17

1.4	The S-Box S2 used in the FSM	18

1.5	Interpretation of the 32-bit words contained in the LFSR as elements of GF(232)	18

ANNEX 2	Implementation options for some operations of the SNOW 3G Algorithm	19

2.1.	The S-Box S1 used in the FSM	19

2.2.	The S-Box S2 used in the FSM	19

2.3.	The functions MUL and DIV used in the LFSR	19

2.4.	Definitions of tables for the FSM	20

2.5.	Definitions of tables for the LFSR	28

ANNEX 3	Figures of the SNOW 3G Algorithm	30

SNOW 3G Algorithm during key initialisation	30

SNOW 3G Algorithm during keystream-generation	31

ANNEX 4	Simulation Program Listing	32

4.1.	Header file	32

4.2.	Code	32

REFERENCES

- 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Security Architecture (3G TS 33.102 version 6.3.0).

- 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Cryptographic Algorithm Requirements; (3G TS 33.105 version 6.0.0).

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 1: UEA2 and UIA2 specifications.

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2. 
Document 2: SNOW 3G specification.

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 3: Implementors’ Test Data.

- Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2.
Document 4: Design Conformance Test Data.

- P. Ekdahl and T. Johansson, “A new version of the stream cipher SNOW”, in Selected Areas in Cryptology (SAC 2002), LNCS 2595, pp. 47–61, Springer-Verlag.

- J. Daemen, V. Rijmen, “The design of Rijndael”, Springer Verlag Series on Information Security and Cryptography, Springer Verlag, 2002, ISBN 3-540-42580-2.

NORMATIVE SECTION

This part of the document contains the normative specification of the SNOW 3G algorithm.

# Outline of the Normative Part of the Document

Section 2 introduces the algorithm and describes the notation used in the subsequent sections.

Section 3 defines the basic components of the algorithm.

Section 4 defines the operation of SNOW 3G.

Section 5 contains tables defining functions used in SNOW 3G.

# Introductory Information

## Introduction

Within the security architecture of the 3GPP system there are standardised algorithms: A confidentiality algorithm UEA2, and an integrity algorithm UIA2.  These algorithms are fully specified in a companion document. Each of these algorithms is based on the SNOW 3G algorithm that is specified here.

SNOW 3G is a word-oriented stream cipher that generates a sequence of 32-bit words under the control of a 128-bit key and a 128-bit initialisation variable. These words can be used to mask the plaintext. First a key initialisation is performed, i.e. the cipher is clocked without producing output, see 4.1. Then with every clock tick it produces a 32-bit word of output, see 4.2.

## Notation

### Radix

We use the prefix 0x to indicate hexadecimal numbers.

### Bit ordering

All data variables in this specification are presented with the most significant bit on the left hand side and the least significant bit on the right hand side. Where a variable is broken down into a number of sub-strings, the left most (most significant) sub-string is numbered 0, the next most significant is numbered 1 and so on through to the least significant.

For example if a 64-bit value X is subdivided into four 16-bit substrings P, Q, R, S we have:

X = 0x0123456789ABCDEF

with

P = 0x0123,  Q = 0x4567,  R = 0x89AB, S = 0xCDEF.

In binary this would be:

X = 0000000100100011010001010110011110001001101010111100110111101111

with 	P = 0000000100100011

Q = 0100010101100111

R = 1000100110101011

S = 1100110111101111

### Conventions

We use the assignment operator ‘=’, as used in several programming languages. When we write

<variable> = <expression>

we mean that <variable> assumes the value that <expression> had before the assignment took place. For instance,

x = x + y + 3

means

(new value of x) becomes (old value of x) + (old value of y) + 3.

### List of Symbols

=	The assignment operator.

	The bitwise exclusive-OR operation.

⊞	Integer addition modulo 232.

||	The concatenation of the two operands.

<<n t	t-bit left shift in an n-bit register.

# Components of SNOW 3G

## Functions used in different Components of SNOW 3G

### MULx

MULx maps 16 bits to 8 bits. Let V and c be 8-bit input values. Then MULx is defined:
If the leftmost (i.e. the most significant) bit of V equals 1, then 
	MULx(V, c) = (V <<8 1)  c,
else 
	MULx(V, c) = V <<8 1.

Example: 
MULx(0x69,0x1B) = 0xC2
MULx(0x96,0x1B) = 0x2C  0x1B = 0x37.

### MULxPOW

MULxPOW maps 16 bits and an positive integer i to 8 bit. Let V and c be 8-bit input values, then MULxPOW(V, i, c) is recursively defined:
If i equals 0, then 
	MULxPOW(V, i, c) = V,
else
	MULxPOW(V, i, c) = MULx(MULxPOW(V, i - 1, c), c).

## Linear Feedback Shift Register (LFSR)

The Linear Feedback Shift Register (LFSR) consists of sixteen stages s0, s1, s2, …, s15 each holding 32 bits.

## Finite State Machine (FSM)

The Finite State Machine (FSM) has three 32-bit registers R1, R2 and R3.

The S-boxes S1 and S2 are used to update the registers R2 and R3.

### The 32x32-bit S-Box S1

The S-Box S1 maps a 32-bit input to a 32-bit output.
Let w = w0 || w1 || w2 || w3 the 32-bit input with w0 the most and w3 the least significant byte. Let S1(w)= r0 || r1 || r2 || r3 with r0 the most and r3 the least significant byte. We use the 8 to 8 bit Rijndael S-Box SR defined in 5.1.

Then r0, r1, r2, r3 are defined as

| r0= | MULx(SR(w0), 0x1B) |  | SR(w1) |  | SR(w2) |  | MULx(SR(w3), 0x1B)  SR(w3), |
| --- | --- | --- | --- | --- | --- | --- | --- |
| r1= | MULx(SR(w0), 0x1B)  SR(w0) |  | MULx(SR(w1), 0x1B) |  | SR(w2) |  | SR(w3), |
| r2= | SR(w0) |  | MULx(SR(w1), 0x1B)  SR(w1) |  | MULx(SR(w2), 0x1B) |  | SR(w3), |
| r3= | SR(w0) |  | SR(w1) |  | MULx(SR(w2), 0x1B)  SR(w2) |  | MULx(SR(w3), 0x1B). |

### The 32x32-bit S-Box S2

The S-Box S2 maps a 32-bit input to a 32-bit output.
Let w = w0 || w1 || w2 || w3 the 32-bit input with w0 the most and w3 the least significant byte. Let S2(w)= r0 || r1 || r2 || r3 with r0 the most and r3 the least significant byte. We use the 8 to 8 bit S-Box SQ defined in 5.2

Then r0, r1, r2, r3 are defined as

| r0= | MULx(SQ(w0), 0x69) |  | SQ(w1) |  | SQ(w2) |  | MULx(SQ(w3), 0x69)  SQ(w3), |
| --- | --- | --- | --- | --- | --- | --- | --- |
| r1= | MULx(SQ(w0), 0x69)  SQ(w0) |  | MULx(SQ(w1), 0x69) |  | SQ(w2) |  | SQ(w3), |
| r2= | SQ(w0) |  | MULx(SQ(w1), 0x69)  SQ(w1) |  | MULx(SQ(w2), 0x69) |  | SQ(w3), |
| r3= | SQ(w0) |  | SQ(w1) |  | MULx(SQ(w2), 0x69)  SQ(w2) |  | MULx(SQ(w3), 0x69). |

## The Clocking Operations

### Clocking the LFSR

The clocking of the LFSR has two different modes of operation, the Initialisation Mode 3.4.4 and the Keystream Mode 3.4.5.
In both modes the functions MUL and DIV are used which are defined in 3.4.2 resp. 3.4.3.

### The function MUL

The function MUL maps 8 bits to 32 bits. Let c be the 8-bit input, then MUL is defined as

MUL(c) =

(MULxPOW(c, 23, 0xA9) || MULxPOW(c, 245, 0xA9) || MULxPOW(c, 48, 0xA9) || MULxPOW(c, 239, 0xA9)).

### The function DIV

The function DIV maps 8 bits to 32 bits. Let c be the 8-bit input, then DIV is defined as

DIV(c) =

(MULxPOW(c, 16, 0xA9) || MULxPOW(c, 39, 0xA9) || MULxPOW(c, 6, 0xA9) || MULxPOW(c, 64, 0xA9)).

### Initialisation Mode

In the Initialisation Mode the LFSR receives a 32-bit input word F, which is the output of the FSM.

Let s0 = s0,0 || s0,1 || s0,2 || s0,3 with s0,0 being the most and s0,3 being the least significant byte of s0.

Let s11 = s11,0 || s11,1 || s11,2 || s11,3 with s11,0 being the most and s11,3 being the least significant byte of s11.

Compute the intermediate value v as

v = (s0,1 || s0,2 || s0,3 || 0x00)  MUL(s0,0)  s2  (0x00 || s11,0 || s11,1 || s11,2)  DIV(s11,3)  F.

Set

s0 = s1,	s1 = s2,	s2 = s3,	s3 = s4,	s4 = s5,	s5 = s6,	s6 = s7,	s7 = s8,	
s8 = s9,	s9 = s10,	s10 = s11,	s11 = s12,	s12 = s13,	s13 = s14,	s14 = s15,	s15 = v.

### Keystream Mode

In the Keystream Mode the LFSR does not receive any input.

Let s0 = s0,0 || s0,1 || s0,2 || s0,3 with s0,0 being the most and s0,3 being the least significant byte of s0.

Let s11 = s11,0 || s11,1 || s11,2 || s11,3 with s11,0 being the most and s11,3 being the least significant byte of s11.

Compute the intermediate value v as

v = (s0,1 || s0,2 || s0,3 || 0x00)  MUL(s0,0)  s2  (0x00 || s11,0 || s11,1 || s11,2)  DIV(s11,3).

Set

s0 = s1,	s1 = s2,	s2 = s3,	s3 = s4,	s4 = s5,	s5 = s6,	s6 = s7,	s7 = s8,	
s8 = s9,	s9 = s10,	s10 = s11,	s11 = s12,	s12 = s13,	s13 = s14,	s14 = s15,	s15 = v.

### Clocking the FSM

The FSM has two input words s15 and s5 from the LFSR.
It produces a 32-bit output word F:

F = (s15 ⊞ R1)  R2

Then the registers are updated. 
Compute the intermediate value r as

r = R2 ⊞ (R3  s5).

Set

R3 = S2(R2),

R2 = S1(R1),

R1 = r.

# Operation of SNOW 3G

## Initialisation

SNOW 3G is initialized with a 128-bit key consisting of four 32-bit words k0, k1, k2, k3 and an 128-bit initialisation variable consisting of four 32-bit words IV0, IV1, IV2, IV3  as follows.

Let 1 be the all-ones word (0xffffffff).

s15 = k3  IV0			s14= k2 			s13 = k1			s12 = k0  IV1

s11 = k3  1			s10= k2  1  IV2	s9 = k1  1  IV3	s8 = k0  1

s7 = k3 				s6= k2			s5 = k1			s4 = k0

s3 = k3  1			s2= k2 1			s1 = k1  1		s0 = k0  1

The FSM is initialised with R1 = R2 = R3 = 0.

Then the cipher runs in a special mode without producing output:

repeat 32-times {

STEP 1: The FSM is clocked (see 3.4.6) producing the 32-bit word F.

STEP 2: Then the LFSR is clocked in Initialisation Mode (see 3.4.4) consuming F.

}

## Generation of Keystream

First the FSM is clocked once, see 3.4.6. The output word of the FSM is discarded.
Then the LFSR is clocked once in Keystream Mode, see 3.4.4.

After that n 32-bit words of keystream are produced:
for t = 1 to n {
STEP 1: The FSM is clocked (see 3.4.6) and produces a 32-bit output word F.
STEP 2: The next keystream word is computed as zt = F  s0.
STEP 3: Then the LFSR is clocked in Keystream Mode, see 3.4.4.
}

# Definition of Tables used in SNOW 3G

### The Rijndael S-box SR

The S-box SR maps 8 bit to 8 bit. Here the input and output is presented in hexadecimal form.

Let x0, x1, y0, y1 be hexadecimal digits with SR(x0 24+x1) = y0 24 + y1, then the cell at the intersection of the x0th row and the x1th column contains the values for y0||y1 in hexadecimal form.

For example SR(42) = SR(0x2A) = 0xE5 = 229.

| x1 x0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 63 | 7C | 77 | 7B | F2 | 6B | 6F | C5 | 30 | 01 | 67 | 2B | FE | D7 | AB | 76 |
| 1 | CA | 82 | C9 | 7D | FA | 59 | 47 | F0 | AD | D4 | A2 | AF | 9C | A4 | 72 | C0 |
| 2 | B7 | FD | 93 | 26 | 36 | 3F | F7 | CC | 34 | A5 | E5 | F1 | 71 | D8 | 31 | 15 |
| 3 | 04 | C7 | 23 | C3 | 18 | 96 | 05 | 9A | 07 | 12 | 80 | E2 | EB | 27 | B2 | 75 |
| 4 | 09 | 83 | 2C | 1A | 1B | 6E | 5A | A0 | 52 | 3B | D6 | B3 | 29 | E3 | 2F | 84 |
| 5 | 53 | D1 | 00 | ED | 20 | FC | B1 | 5B | 6A | CB | BE | 39 | 4A | 4C | 58 | CF |
| 6 | D0 | EF | AA | FB | 43 | 4D | 33 | 85 | 45 | F9 | 02 | 7F | 50 | 3C | 9F | A8 |
| 7 | 51 | A3 | 40 | 8F | 92 | 9D | 38 | F5 | BC | B6 | DA | 21 | 10 | FF | F3 | D2 |
| 8 | CD | 0C | 13 | EC | 5F | 97 | 44 | 17 | C4 | A7 | 7E | 3D | 64 | 5D | 19 | 73 |
| 9 | 60 | 81 | 4F | DC | 22 | 2A | 90 | 88 | 46 | EE | B8 | 14 | DE | 5E | 0B | DB |
| A | E0 | 32 | 3A | 0A | 49 | 06 | 24 | 5C | C2 | D3 | AC | 62 | 91 | 95 | E4 | 79 |
| B | E7 | C8 | 37 | 6D | 8D | D5 | 4E | A9 | 6C | 56 | F4 | EA | 65 | 7A | AE | 08 |
| C | BA | 78 | 25 | 2E | 1C | A6 | B4 | C6 | E8 | DD | 74 | 1F | 4B | BD | 8B | 8A |
| D | 70 | 3E | B5 | 66 | 48 | 03 | F6 | 0E | 61 | 35 | 57 | B9 | 86 | C1 | 1D | 9E |
| E | E1 | F8 | 98 | 11 | 69 | D9 | 8E | 94 | 9B | 1E | 87 | E9 | CE | 55 | 28 | DF |
| F | 8C | A1 | 89 | 0D | BF | E6 | 42 | 68 | 41 | 99 | 2D | 0F | B0 | 54 | BB | 16 |

Table 1: Rijndael S-Box

### The S-box SQ

The S-box SQ maps 8 bit to 8 bit. Here the input is presented in hexadecimal form.

Let x0, x1, y0, y1 be hexadecimal digits with SQ(x0 24+x1) = y0 24 + y1, then the cell at the intersection of the x0th row and the x1th column contains the values for y0||y1 in hexadecimal form.

For example SQ(42) = SQ(0x2A) = 0xAC = 172.

| x1 x0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 25 | 24 | 73 | 67 | D7 | AE | 5C | 30 | A4 | EE | 6E | CB | 7D | B5 | 82 | DB |
| 1 | E4 | 8E | 48 | 49 | 4F | 5D | 6A | 78 | 70 | 88 | E8 | 5F | 5E | 84 | 65 | E2 |
| 2 | D8 | E9 | CC | ED | 40 | 2F | 11 | 28 | 57 | D2 | AC | E3 | 4A | 15 | 1B | B9 |
| 3 | B2 | 80 | 85 | A6 | 2E | 02 | 47 | 29 | 07 | 4B | 0E | C1 | 51 | AA | 89 | D4 |
| 4 | CA | 01 | 46 | B3 | EF | DD | 44 | 7B | C2 | 7F | BE | C3 | 9F | 20 | 4C | 64 |
| 5 | 83 | A2 | 68 | 42 | 13 | B4 | 41 | CD | BA | C6 | BB | 6D | 4D | 71 | 21 | F4 |
| 6 | 8D | B0 | E5 | 93 | FE | 8F | E6 | CF | 43 | 45 | 31 | 22 | 37 | 36 | 96 | FA |
| 7 | BC | 0F | 08 | 52 | 1D | 55 | 1A | C5 | 4E | 23 | 69 | 7A | 92 | FF | 5B | 5A |
| 8 | EB | 9A | 1C | A9 | D1 | 7E | 0D | FC | 50 | 8A | B6 | 62 | F5 | 0A | F8 | DC |
| 9 | 03 | 3C | 0C | 39 | F1 | B8 | F3 | 3D | F2 | D5 | 97 | 66 | 81 | 32 | A0 | 00 |
| A | 06 | CE | F6 | EA | B7 | 17 | F7 | 8C | 79 | D6 | A7 | BF | 8B | 3F | 1F | 53 |
| B | 63 | 75 | 35 | 2C | 60 | FD | 27 | D3 | 94 | A5 | 7C | A1 | 05 | 58 | 2D | BD |
| C | D9 | C7 | AF | 6B | 54 | 0B | E0 | 38 | 04 | C8 | 9D | E7 | 14 | B1 | 87 | 9C |
| D | DF | 6F | F9 | DA | 2A | C4 | 59 | 16 | 74 | 91 | AB | 26 | 61 | 76 | 34 | 2B |
| E | AD | 99 | FB | 72 | EC | 33 | 12 | DE | 98 | 3B | C0 | 9B | 3E | 18 | 10 | 3A |
| F | 56 | E1 | 77 | C9 | 1E | 9E | 95 | A3 | 90 | 19 | A8 | 6C | 09 | D0 | F0 | 86 |

Table 2: S-Box SQ

INFORMATIVE SECTION

This part of the document is purely informative and does not form part of the normative specification of SNOW 3G.

# Remarks about the mathematical background of some operations of the SNOW 3G Algorithm

## MULx and MULxPOW

The function MULx (see 3.1.1) corresponds to the multiplication with the primitive element of an extension of degree 8 of GF(2). Describe GF(28) as GF(2)() with  being the root of the irreducible polynomial 
x8 + c0x7 + c1x6 + c2x5 + c3x4 + c4x3 + c5x2 + c6x + c7. 
Define the 8-bit value c as c = c0 || c1 || c2 || c3 || c4 || c5 || c6 || c7 with c0 the most and c7 the least significant bit. Then for an element V of GF(28) the result of MULx(V, c) corresponds to V.

MULxPOW (see 3.1.2) corresponds to the multiplication with the primitive element raised to the power of an positive integer i.
With the definition of above MULxPOW(V, i, c) corresponds to Vi in GF(2)().

## The S-Box S1 used in the FSM

The S-Box S1 is based on the round function of Rijndael [8].

Consider an 32-bit input word w = w0 || w1 || w2 || w3 where w0 is the most and w3 the least significant byte.

Here the bytes are interpreted as elements of GF(28) defined by the polynomial

x8 + x4 + x3 + x + 1.

Hence wi = wi0 || wi1 || wi2 || wi3 || wi4 || wi5 || wi6 || wi7 with wi0 the most and wi7 the least significant bit is interpreted as wi0 x7 + wi1 x6 + wi2 x5 + wi3 x4 + wi4 x3 + wi5 x2 + wi6 x1 + wi7.

Then the output S1(w) = r = r0 || r1 || r2 || r3 with r0 the most r1 the least significant byte is defined as

r0 =	(x + 1)	SR(w3)	+		SR(w2)	+		SR(w1)	+	x	SR(w0),

r1 =		SR(w3)	+		SR(w2)	+	x	SR(w1)	+	(x + 1)	SR(w0),

r2 =		SR(w3)	+	x	SR(w2)	+	(x + 1)	SR(w1)	+		SR(w0),

r3 =	x	SR(w3)	+	(x + 1)	SR(w2)	+		SR(w1)	+		SR(w0).

Note: The notation used here differs from the notation in [7].

## The S-Box SQ used in the S-Box S2

The S-Box SQ is constructed using the Dickson polynomial 
g49(x) = x  x9  x13  x15  x33  x41  x45  x47 x49. 
For an 8-bit input x in GF(28) defined by the polynomial x8 + x6 + x5 + x3 + 1 the 8-bit output of SQ corresponds to g49(x)  0x25.

## The S-Box S2 used in the FSM

The S-Box S2 is based on the S-box SQ.

Consider an 32-bit input word w = w0 || w1 || w2 || w3 where w0 is the most and w3 the least significant byte.

Here the bytes are interpreted as elements of GF(28) defined by the polynomial

x8 + x6 + x5 + x3 + 1.

Hence wi = wi0 || wi1 || wi2 || wi3 || wi4 || wi5 || wi6 || wi7 with wi0 the most and wi7 the least significant bit is interpreted as wi0 x7 + wi1 x6 + wi2 x5 + wi3 x4 + wi4 x3 + wi5 x2 + wi6 x1 + wi7.

Then the output S2(w) = r = r0 || r1 || r2 || r3 with r0 the most r1 the least significant byte is defined as

r0 =	(x + 1)	SQ(w3)	+		SQ(w2)	+		SQ(w1)	+	x	SQ(w0),

r1 =		SQ(w3)	+		SQ(w2)	+	x	SQ(w1)	+	(x + 1)	SQ(w0),

r2 =		SQ(w3)	+	x	SQ(w2)	+	(x + 1)	SQ(w1)	+		SQ(w0),

r3 =	x	SQ(w3)	+	(x + 1)	SQ(w2)	+		SQ(w1)	+		SQ(w0).

## Interpretation of the 32-bit words contained in the LFSR as elements of GF(232)

The 32-bit words can be interpreted as elements of the Galois Field GF(232). 
Let  be a root of the irreducible GF(2)[x]-polynomial x8+x7+x5+x3+1.
The elements of GF(28) are expressed on the base 7, 6, 5, 4, 3, 2, , 1.
Let  be a root of the irreducible GF(28)[x] polynomial x4 + 23x3 + 245x2 + 48x + 239. 
The elements of GF(232) are expressed on the base 3, 2, , 1.
Hence if a 32-bit word w is stored in 4 bytes b0, b1, b3, b3 and each byte bi consists of the bits bi0,bi1, … ,bi7, then this word can be interpreted as

w = b03 + b12 + b2  + b3  =

(b007 + b016 + b025 + b034 + b043 + b052 + b06 + b07) 3 +

(b107 + b116 + b125 + b134 + b143 + b152 + b16 + b17) 2 +

(b207 + b216 + b225 + b234 + b243 + b252 + b26 + b27)  +

b307 + b316 + b325 + b334 + b343 + b352 + b36 + b37

With these definitions multiplication of w with  is equal to a byte shift of w to the left and an XOR with the result of MUL.

Similarly a multiplication with -1 can be implemented as a byte shift to the right and an  XOR with the result of DIV.

# Implementation options for some operations of the SNOW 3G Algorithm

In this Annex some alternative definitions are given for some operations of SNOW 3G.

## The S-Box S1 used in the FSM

Consider an 32-bit input word w = w0 || w1 || w2 || w3 where w0 is the most and w3 the least significant byte.
In order to compute S1(w) we use 4 table lookups from the tables S1_T0, S1_T1, S1_T2 and S1_T3 defined in 1.4. Each of these tables maps 8 bits to 32 bits.

Then S1(w) is computed according to

S1(w) = S1_T0(w3)  S1_T1(w2)  S1_T2(w1)  S1_T3(w0).

Note: The notation used here differs from the notation in [7].

## The S-Box S2 used in the FSM

Consider an 32-bit input word w = w0 || w1 || w2 || w3 where w0 is the most and w3 the least significant byte.
In order to compute S2(w) we use 4 table lookups from the tables S2_T0, S2_T1, S2_T2 and S2_T3 defined in 1.4. Each of these tables maps 8 bits to 32 bits.

Then S2(w) is computed according to

S2(w) = S2_T0(w3)  S2_T1(w2)  S2_T2(w1)  S2_T3(w0).

## The functions MUL and DIV used in the LFSR

The functions MUL and DIV can be implemented as a table lookup from the tables MULalpha and DIValpha defined in 1.5.

## Definitions of tables for the FSM

S1_T0:

| 0x00 | 0xA56363C6 | 0x847C7CF8 | 0x997777EE | 0x8D7B7BF6 | 0xDF2F2FF | 0xBD6B6BD6 | 0xB16F6FDE | 0x54C5C591 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x08 | 0x50303060 | 0x3010102 | 0xA96767CE | 0x7D2B2B56 | 0x19FEFEE7 | 0x62D7D7B5 | 0xE6ABAB4D | 0x9A7676EC |
| 0x10 | 0x45CACA8F | 0x9D82821F | 0x40C9C989 | 0x877D7DFA | 0x15FAFAEF | 0xEB5959B2 | 0xC947478E | 0xBF0F0FB |
| 0x18 | 0xECADAD41 | 0x67D4D4B3 | 0xFDA2A25F | 0xEAAFAF45 | 0xBF9C9C23 | 0xF7A4A453 | 0x967272E4 | 0x5BC0C09B |
| 0x20 | 0xC2B7B775 | 0x1CFDFDE1 | 0xAE93933D | 0x6A26264C | 0x5A36366C | 0x413F3F7E | 0x2F7F7F5 | 0x4FCCCC83 |
| 0x28 | 0x5C343468 | 0xF4A5A551 | 0x34E5E5D1 | 0x8F1F1F9 | 0x937171E2 | 0x73D8D8AB | 0x53313162 | 0x3F15152A |
| 0x30 | 0xC040408 | 0x52C7C795 | 0x65232346 | 0x5EC3C39D | 0x28181830 | 0xA1969637 | 0xF05050A | 0xB59A9A2F |
| 0x38 | 0x907070E | 0x36121224 | 0x9B80801B | 0x3DE2E2DF | 0x26EBEBCD | 0x6927274E | 0xCDB2B27F | 0x9F7575EA |
| 0x40 | 0x1B090912 | 0x9E83831D | 0x742C2C58 | 0x2E1A1A34 | 0x2D1B1B36 | 0xB26E6EDC | 0xEE5A5AB4 | 0xFBA0A05B |
| 0x48 | 0xF65252A4 | 0x4D3B3B76 | 0x61D6D6B7 | 0xCEB3B37D | 0x7B292952 | 0x3EE3E3DD | 0x712F2F5E | 0x97848413 |
| 0x50 | 0xF55353A6 | 0x68D1D1B9 | 0x0 | 0x2CEDEDC1 | 0x60202040 | 0x1FFCFCE3 | 0xC8B1B179 | 0xED5B5BB6 |
| 0x58 | 0xBE6A6AD4 | 0x46CBCB8D | 0xD9BEBE67 | 0x4B393972 | 0xDE4A4A94 | 0xD44C4C98 | 0xE85858B0 | 0x4ACFCF85 |
| 0x60 | 0x6BD0D0BB | 0x2AEFEFC5 | 0xE5AAAA4F | 0x16FBFBED | 0xC5434386 | 0xD74D4D9A | 0x55333366 | 0x94858511 |
| 0x68 | 0xCF45458A | 0x10F9F9E9 | 0x6020204 | 0x817F7FFE | 0xF05050A0 | 0x443C3C78 | 0xBA9F9F25 | 0xE3A8A84B |
| 0x70 | 0xF35151A2 | 0xFEA3A35D | 0xC0404080 | 0x8A8F8F05 | 0xAD92923F | 0xBC9D9D21 | 0x48383870 | 0x4F5F5F1 |
| 0x78 | 0xDFBCBC63 | 0xC1B6B677 | 0x75DADAAF | 0x63212142 | 0x30101020 | 0x1AFFFFE5 | 0xEF3F3FD | 0x6DD2D2BF |
| 0x80 | 0x4CCDCD81 | 0x140C0C18 | 0x35131326 | 0x2FECECC3 | 0xE15F5FBE | 0xA2979735 | 0xCC444488 | 0x3917172E |
| 0x88 | 0x57C4C493 | 0xF2A7A755 | 0x827E7EFC | 0x473D3D7A | 0xAC6464C8 | 0xE75D5DBA | 0x2B191932 | 0x957373E6 |
| 0x90 | 0xA06060C0 | 0x98818119 | 0xD14F4F9E | 0x7FDCDCA3 | 0x66222244 | 0x7E2A2A54 | 0xAB90903B | 0x8388880B |
| 0x98 | 0xCA46468C | 0x29EEEEC7 | 0xD3B8B86B | 0x3C141428 | 0x79DEDEA7 | 0xE25E5EBC | 0x1D0B0B16 | 0x76DBDBAD |
| 0xA0 | 0x3BE0E0DB | 0x56323264 | 0x4E3A3A74 | 0x1E0A0A14 | 0xDB494992 | 0xA06060C | 0x6C242448 | 0xE45C5CB8 |
| 0xA8 | 0x5DC2C29F | 0x6ED3D3BD | 0xEFACAC43 | 0xA66262C4 | 0xA8919139 | 0xA4959531 | 0x37E4E4D3 | 0x8B7979F2 |
| 0xB0 | 0x32E7E7D5 | 0x43C8C88B | 0x5937376E | 0xB76D6DDA | 0x8C8D8D01 | 0x64D5D5B1 | 0xD24E4E9C | 0xE0A9A949 |
| 0xB8 | 0xB46C6CD8 | 0xFA5656AC | 0x7F4F4F3 | 0x25EAEACF | 0xAF6565CA | 0x8E7A7AF4 | 0xE9AEAE47 | 0x18080810 |
| 0xC0 | 0xD5BABA6F | 0x887878F0 | 0x6F25254A | 0x722E2E5C | 0x241C1C38 | 0xF1A6A657 | 0xC7B4B473 | 0x51C6C697 |
| 0xC8 | 0x23E8E8CB | 0x7CDDDDA1 | 0x9C7474E8 | 0x211F1F3E | 0xDD4B4B96 | 0xDCBDBD61 | 0x868B8B0D | 0x858A8A0F |
| 0xD0 | 0x907070E0 | 0x423E3E7C | 0xC4B5B571 | 0xAA6666CC | 0xD8484890 | 0x5030306 | 0x1F6F6F7 | 0x120E0E1C |
| 0xD8 | 0xA36161C2 | 0x5F35356A | 0xF95757AE | 0xD0B9B969 | 0x91868617 | 0x58C1C199 | 0x271D1D3A | 0xB99E9E27 |
| 0xE0 | 0x38E1E1D9 | 0x13F8F8EB | 0xB398982B | 0x33111122 | 0xBB6969D2 | 0x70D9D9A9 | 0x898E8E07 | 0xA7949433 |
| 0xE8 | 0xB69B9B2D | 0x221E1E3C | 0x92878715 | 0x20E9E9C9 | 0x49CECE87 | 0xFF5555AA | 0x78282850 | 0x7ADFDFA5 |
| 0xF0 | 0x8F8C8C03 | 0xF8A1A159 | 0x80898909 | 0x170D0D1A | 0xDABFBF65 | 0x31E6E6D7 | 0xC6424284 | 0xB86868D0 |
| 0xF8 | 0xC3414182 | 0xB0999929 | 0x772D2D5A | 0x110F0F1E | 0xCBB0B07B | 0xFC5454A8 | 0xD6BBBB6D | 0x3A16162C |

Table 3: S1_T0

For example S1_T0(2A) = 0x34E5E5D1.

S1_T1:

| 0x00 | 0x6363C6A5 | 0x7C7CF884 | 0x7777EE99 | 0x7B7BF68D | 0xF2F2FF0D | 0x6B6BD6BD | 0x6F6FDEB1 | 0xC5C59154 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x08 | 0x30306050 | 0x1010203 | 0x6767CEA9 | 0x2B2B567D | 0xFEFEE719 | 0xD7D7B562 | 0xABAB4DE6 | 0x7676EC9A |
| 0x10 | 0xCACA8F45 | 0x82821F9D | 0xC9C98940 | 0x7D7DFA87 | 0xFAFAEF15 | 0x5959B2EB | 0x47478EC9 | 0xF0F0FB0B |
| 0x18 | 0xADAD41EC | 0xD4D4B367 | 0xA2A25FFD | 0xAFAF45EA | 0x9C9C23BF | 0xA4A453F7 | 0x7272E496 | 0xC0C09B5B |
| 0x20 | 0xB7B775C2 | 0xFDFDE11C | 0x93933DAE | 0x26264C6A | 0x36366C5A | 0x3F3F7E41 | 0xF7F7F502 | 0xCCCC834F |
| 0x28 | 0x3434685C | 0xA5A551F4 | 0xE5E5D134 | 0xF1F1F908 | 0x7171E293 | 0xD8D8AB73 | 0x31316253 | 0x15152A3F |
| 0x30 | 0x404080C | 0xC7C79552 | 0x23234665 | 0xC3C39D5E | 0x18183028 | 0x969637A1 | 0x5050A0F | 0x9A9A2FB5 |
| 0x38 | 0x7070E09 | 0x12122436 | 0x80801B9B | 0xE2E2DF3D | 0xEBEBCD26 | 0x27274E69 | 0xB2B27FCD | 0x7575EA9F |
| 0x40 | 0x909121B | 0x83831D9E | 0x2C2C5874 | 0x1A1A342E | 0x1B1B362D | 0x6E6EDCB2 | 0x5A5AB4EE | 0xA0A05BFB |
| 0x48 | 0x5252A4F6 | 0x3B3B764D | 0xD6D6B761 | 0xB3B37DCE | 0x2929527B | 0xE3E3DD3E | 0x2F2F5E71 | 0x84841397 |
| 0x50 | 0x5353A6F5 | 0xD1D1B968 | 0x0 | 0xEDEDC12C | 0x20204060 | 0xFCFCE31F | 0xB1B179C8 | 0x5B5BB6ED |
| 0x58 | 0x6A6AD4BE | 0xCBCB8D46 | 0xBEBE67D9 | 0x3939724B | 0x4A4A94DE | 0x4C4C98D4 | 0x5858B0E8 | 0xCFCF854A |
| 0x60 | 0xD0D0BB6B | 0xEFEFC52A | 0xAAAA4FE5 | 0xFBFBED16 | 0x434386C5 | 0x4D4D9AD7 | 0x33336655 | 0x85851194 |
| 0x68 | 0x45458ACF | 0xF9F9E910 | 0x2020406 | 0x7F7FFE81 | 0x5050A0F0 | 0x3C3C7844 | 0x9F9F25BA | 0xA8A84BE3 |
| 0x70 | 0x5151A2F3 | 0xA3A35DFE | 0x404080C0 | 0x8F8F058A | 0x92923FAD | 0x9D9D21BC | 0x38387048 | 0xF5F5F104 |
| 0x78 | 0xBCBC63DF | 0xB6B677C1 | 0xDADAAF75 | 0x21214263 | 0x10102030 | 0xFFFFE51A | 0xF3F3FD0E | 0xD2D2BF6D |
| 0x80 | 0xCDCD814C | 0xC0C1814 | 0x13132635 | 0xECECC32F | 0x5F5FBEE1 | 0x979735A2 | 0x444488CC | 0x17172E39 |
| 0x88 | 0xC4C49357 | 0xA7A755F2 | 0x7E7EFC82 | 0x3D3D7A47 | 0x6464C8AC | 0x5D5DBAE7 | 0x1919322B | 0x7373E695 |
| 0x90 | 0x6060C0A0 | 0x81811998 | 0x4F4F9ED1 | 0xDCDCA37F | 0x22224466 | 0x2A2A547E | 0x90903BAB | 0x88880B83 |
| 0x98 | 0x46468CCA | 0xEEEEC729 | 0xB8B86BD3 | 0x1414283C | 0xDEDEA779 | 0x5E5EBCE2 | 0xB0B161D | 0xDBDBAD76 |
| 0xA0 | 0xE0E0DB3B | 0x32326456 | 0x3A3A744E | 0xA0A141E | 0x494992DB | 0x6060C0A | 0x2424486C | 0x5C5CB8E4 |
| 0xA8 | 0xC2C29F5D | 0xD3D3BD6E | 0xACAC43EF | 0x6262C4A6 | 0x919139A8 | 0x959531A4 | 0xE4E4D337 | 0x7979F28B |
| 0xB0 | 0xE7E7D532 | 0xC8C88B43 | 0x37376E59 | 0x6D6DDAB7 | 0x8D8D018C | 0xD5D5B164 | 0x4E4E9CD2 | 0xA9A949E0 |
| 0xB8 | 0x6C6CD8B4 | 0x5656ACFA | 0xF4F4F307 | 0xEAEACF25 | 0x6565CAAF | 0x7A7AF48E | 0xAEAE47E9 | 0x8081018 |
| 0xC0 | 0xBABA6FD5 | 0x7878F088 | 0x25254A6F | 0x2E2E5C72 | 0x1C1C3824 | 0xA6A657F1 | 0xB4B473C7 | 0xC6C69751 |
| 0xC8 | 0xE8E8CB23 | 0xDDDDA17C | 0x7474E89C | 0x1F1F3E21 | 0x4B4B96DD | 0xBDBD61DC | 0x8B8B0D86 | 0x8A8A0F85 |
| 0xD0 | 0x7070E090 | 0x3E3E7C42 | 0xB5B571C4 | 0x6666CCAA | 0x484890D8 | 0x3030605 | 0xF6F6F701 | 0xE0E1C12 |
| 0xD8 | 0x6161C2A3 | 0x35356A5F | 0x5757AEF9 | 0xB9B969D0 | 0x86861791 | 0xC1C19958 | 0x1D1D3A27 | 0x9E9E27B9 |
| 0xE0 | 0xE1E1D938 | 0xF8F8EB13 | 0x98982BB3 | 0x11112233 | 0x6969D2BB | 0xD9D9A970 | 0x8E8E0789 | 0x949433A7 |
| 0xE8 | 0x9B9B2DB6 | 0x1E1E3C22 | 0x87871592 | 0xE9E9C920 | 0xCECE8749 | 0x5555AAFF | 0x28285078 | 0xDFDFA57A |
| 0xF0 | 0x8C8C038F | 0xA1A159F8 | 0x89890980 | 0xD0D1A17 | 0xBFBF65DA | 0xE6E6D731 | 0x424284C6 | 0x6868D0B8 |
| 0xF8 | 0x414182C3 | 0x999929B0 | 0x2D2D5A77 | 0xF0F1E11 | 0xB0B07BCB | 0x5454A8FC | 0xBBBB6DD6 | 0x16162C3A |

Table 4: S1_T1

S1_T2:

| 0x00 | 0x63C6A563 | 0x7CF8847C | 0x77EE9977 | 0x7BF68D7B | 0xF2FF0DF2 | 0x6BD6BD6B | 0x6FDEB16F | 0xC59154C5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x08 | 0x30605030 | 0x1020301 | 0x67CEA967 | 0x2B567D2B | 0xFEE719FE | 0xD7B562D7 | 0xAB4DE6AB | 0x76EC9A76 |
| 0x10 | 0xCA8F45CA | 0x821F9D82 | 0xC98940C9 | 0x7DFA877D | 0xFAEF15FA | 0x59B2EB59 | 0x478EC947 | 0xF0FB0BF0 |
| 0x18 | 0xAD41ECAD | 0xD4B367D4 | 0xA25FFDA2 | 0xAF45EAAF | 0x9C23BF9C | 0xA453F7A4 | 0x72E49672 | 0xC09B5BC0 |
| 0x20 | 0xB775C2B7 | 0xFDE11CFD | 0x933DAE93 | 0x264C6A26 | 0x366C5A36 | 0x3F7E413F | 0xF7F502F7 | 0xCC834FCC |
| 0x28 | 0x34685C34 | 0xA551F4A5 | 0xE5D134E5 | 0xF1F908F1 | 0x71E29371 | 0xD8AB73D8 | 0x31625331 | 0x152A3F15 |
| 0x30 | 0x4080C04 | 0xC79552C7 | 0x23466523 | 0xC39D5EC3 | 0x18302818 | 0x9637A196 | 0x50A0F05 | 0x9A2FB59A |
| 0x38 | 0x70E0907 | 0x12243612 | 0x801B9B80 | 0xE2DF3DE2 | 0xEBCD26EB | 0x274E6927 | 0xB27FCDB2 | 0x75EA9F75 |
| 0x40 | 0x9121B09 | 0x831D9E83 | 0x2C58742C | 0x1A342E1A | 0x1B362D1B | 0x6EDCB26E | 0x5AB4EE5A | 0xA05BFBA0 |
| 0x48 | 0x52A4F652 | 0x3B764D3B | 0xD6B761D6 | 0xB37DCEB3 | 0x29527B29 | 0xE3DD3EE3 | 0x2F5E712F | 0x84139784 |
| 0x50 | 0x53A6F553 | 0xD1B968D1 | 0x0 | 0xEDC12CED | 0x20406020 | 0xFCE31FFC | 0xB179C8B1 | 0x5BB6ED5B |
| 0x58 | 0x6AD4BE6A | 0xCB8D46CB | 0xBE67D9BE | 0x39724B39 | 0x4A94DE4A | 0x4C98D44C | 0x58B0E858 | 0xCF854ACF |
| 0x60 | 0xD0BB6BD0 | 0xEFC52AEF | 0xAA4FE5AA | 0xFBED16FB | 0x4386C543 | 0x4D9AD74D | 0x33665533 | 0x85119485 |
| 0x68 | 0x458ACF45 | 0xF9E910F9 | 0x2040602 | 0x7FFE817F | 0x50A0F050 | 0x3C78443C | 0x9F25BA9F | 0xA84BE3A8 |
| 0x70 | 0x51A2F351 | 0xA35DFEA3 | 0x4080C040 | 0x8F058A8F | 0x923FAD92 | 0x9D21BC9D | 0x38704838 | 0xF5F104F5 |
| 0x78 | 0xBC63DFBC | 0xB677C1B6 | 0xDAAF75DA | 0x21426321 | 0x10203010 | 0xFFE51AFF | 0xF3FD0EF3 | 0xD2BF6DD2 |
| 0x80 | 0xCD814CCD | 0xC18140C | 0x13263513 | 0xECC32FEC | 0x5FBEE15F | 0x9735A297 | 0x4488CC44 | 0x172E3917 |
| 0x88 | 0xC49357C4 | 0xA755F2A7 | 0x7EFC827E | 0x3D7A473D | 0x64C8AC64 | 0x5DBAE75D | 0x19322B19 | 0x73E69573 |
| 0x90 | 0x60C0A060 | 0x81199881 | 0x4F9ED14F | 0xDCA37FDC | 0x22446622 | 0x2A547E2A | 0x903BAB90 | 0x880B8388 |
| 0x98 | 0x468CCA46 | 0xEEC729EE | 0xB86BD3B8 | 0x14283C14 | 0xDEA779DE | 0x5EBCE25E | 0xB161D0B | 0xDBAD76DB |
| 0xA0 | 0xE0DB3BE0 | 0x32645632 | 0x3A744E3A | 0xA141E0A | 0x4992DB49 | 0x60C0A06 | 0x24486C24 | 0x5CB8E45C |
| 0xA8 | 0xC29F5DC2 | 0xD3BD6ED3 | 0xAC43EFAC | 0x62C4A662 | 0x9139A891 | 0x9531A495 | 0xE4D337E4 | 0x79F28B79 |
| 0xB0 | 0xE7D532E7 | 0xC88B43C8 | 0x376E5937 | 0x6DDAB76D | 0x8D018C8D | 0xD5B164D5 | 0x4E9CD24E | 0xA949E0A9 |
| 0xB8 | 0x6CD8B46C | 0x56ACFA56 | 0xF4F307F4 | 0xEACF25EA | 0x65CAAF65 | 0x7AF48E7A | 0xAE47E9AE | 0x8101808 |
| 0xC0 | 0xBA6FD5BA | 0x78F08878 | 0x254A6F25 | 0x2E5C722E | 0x1C38241C | 0xA657F1A6 | 0xB473C7B4 | 0xC69751C6 |
| 0xC8 | 0xE8CB23E8 | 0xDDA17CDD | 0x74E89C74 | 0x1F3E211F | 0x4B96DD4B | 0xBD61DCBD | 0x8B0D868B | 0x8A0F858A |
| 0xD0 | 0x70E09070 | 0x3E7C423E | 0xB571C4B5 | 0x66CCAA66 | 0x4890D848 | 0x3060503 | 0xF6F701F6 | 0xE1C120E |
| 0xD8 | 0x61C2A361 | 0x356A5F35 | 0x57AEF957 | 0xB969D0B9 | 0x86179186 | 0xC19958C1 | 0x1D3A271D | 0x9E27B99E |
| 0xE0 | 0xE1D938E1 | 0xF8EB13F8 | 0x982BB398 | 0x11223311 | 0x69D2BB69 | 0xD9A970D9 | 0x8E07898E | 0x9433A794 |
| 0xE8 | 0x9B2DB69B | 0x1E3C221E | 0x87159287 | 0xE9C920E9 | 0xCE8749CE | 0x55AAFF55 | 0x28507828 | 0xDFA57ADF |
| 0xF0 | 0x8C038F8C | 0xA159F8A1 | 0x89098089 | 0xD1A170D | 0xBF65DABF | 0xE6D731E6 | 0x4284C642 | 0x68D0B868 |
| 0xF8 | 0x4182C341 | 0x9929B099 | 0x2D5A772D | 0xF1E110F | 0xB07BCBB0 | 0x54A8FC54 | 0xBB6DD6BB | 0x162C3A16 |

Table 5: S1_T2

S1_T3:

| 0x00 | 0xC6A56363 | 0xF8847C7C | 0xEE997777 | 0xF68D7B7B | 0xFF0DF2F2 | 0xD6BD6B6B | 0xDEB16F6F | 0x9154C5C5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x08 | 0x60503030 | 0x2030101 | 0xCEA96767 | 0x567D2B2B | 0xE719FEFE | 0xB562D7D7 | 0x4DE6ABAB | 0xEC9A7676 |
| 0x10 | 0x8F45CACA | 0x1F9D8282 | 0x8940C9C9 | 0xFA877D7D | 0xEF15FAFA | 0xB2EB5959 | 0x8EC94747 | 0xFB0BF0F0 |
| 0x18 | 0x41ECADAD | 0xB367D4D4 | 0x5FFDA2A2 | 0x45EAAFAF | 0x23BF9C9C | 0x53F7A4A4 | 0xE4967272 | 0x9B5BC0C0 |
| 0x20 | 0x75C2B7B7 | 0xE11CFDFD | 0x3DAE9393 | 0x4C6A2626 | 0x6C5A3636 | 0x7E413F3F | 0xF502F7F7 | 0x834FCCCC |
| 0x28 | 0x685C3434 | 0x51F4A5A5 | 0xD134E5E5 | 0xF908F1F1 | 0xE2937171 | 0xAB73D8D8 | 0x62533131 | 0x2A3F1515 |
| 0x30 | 0x80C0404 | 0x9552C7C7 | 0x46652323 | 0x9D5EC3C3 | 0x30281818 | 0x37A19696 | 0xA0F0505 | 0x2FB59A9A |
| 0x38 | 0xE090707 | 0x24361212 | 0x1B9B8080 | 0xDF3DE2E2 | 0xCD26EBEB | 0x4E692727 | 0x7FCDB2B2 | 0xEA9F7575 |
| 0x40 | 0x121B0909 | 0x1D9E8383 | 0x58742C2C | 0x342E1A1A | 0x362D1B1B | 0xDCB26E6E | 0xB4EE5A5A | 0x5BFBA0A0 |
| 0x48 | 0xA4F65252 | 0x764D3B3B | 0xB761D6D6 | 0x7DCEB3B3 | 0x527B2929 | 0xDD3EE3E3 | 0x5E712F2F | 0x13978484 |
| 0x50 | 0xA6F55353 | 0xB968D1D1 | 0x0 | 0xC12CEDED | 0x40602020 | 0xE31FFCFC | 0x79C8B1B1 | 0xB6ED5B5B |
| 0x58 | 0xD4BE6A6A | 0x8D46CBCB | 0x67D9BEBE | 0x724B3939 | 0x94DE4A4A | 0x98D44C4C | 0xB0E85858 | 0x854ACFCF |
| 0x60 | 0xBB6BD0D0 | 0xC52AEFEF | 0x4FE5AAAA | 0xED16FBFB | 0x86C54343 | 0x9AD74D4D | 0x66553333 | 0x11948585 |
| 0x68 | 0x8ACF4545 | 0xE910F9F9 | 0x4060202 | 0xFE817F7F | 0xA0F05050 | 0x78443C3C | 0x25BA9F9F | 0x4BE3A8A8 |
| 0x70 | 0xA2F35151 | 0x5DFEA3A3 | 0x80C04040 | 0x58A8F8F | 0x3FAD9292 | 0x21BC9D9D | 0x70483838 | 0xF104F5F5 |
| 0x78 | 0x63DFBCBC | 0x77C1B6B6 | 0xAF75DADA | 0x42632121 | 0x20301010 | 0xE51AFFFF | 0xFD0EF3F3 | 0xBF6DD2D2 |
| 0x80 | 0x814CCDCD | 0x18140C0C | 0x26351313 | 0xC32FECEC | 0xBEE15F5F | 0x35A29797 | 0x88CC4444 | 0x2E391717 |
| 0x88 | 0x9357C4C4 | 0x55F2A7A7 | 0xFC827E7E | 0x7A473D3D | 0xC8AC6464 | 0xBAE75D5D | 0x322B1919 | 0xE6957373 |
| 0x90 | 0xC0A06060 | 0x19988181 | 0x9ED14F4F | 0xA37FDCDC | 0x44662222 | 0x547E2A2A | 0x3BAB9090 | 0xB838888 |
| 0x98 | 0x8CCA4646 | 0xC729EEEE | 0x6BD3B8B8 | 0x283C1414 | 0xA779DEDE | 0xBCE25E5E | 0x161D0B0B | 0xAD76DBDB |
| 0xA0 | 0xDB3BE0E0 | 0x64563232 | 0x744E3A3A | 0x141E0A0A | 0x92DB4949 | 0xC0A0606 | 0x486C2424 | 0xB8E45C5C |
| 0xA8 | 0x9F5DC2C2 | 0xBD6ED3D3 | 0x43EFACAC | 0xC4A66262 | 0x39A89191 | 0x31A49595 | 0xD337E4E4 | 0xF28B7979 |
| 0xB0 | 0xD532E7E7 | 0x8B43C8C8 | 0x6E593737 | 0xDAB76D6D | 0x18C8D8D | 0xB164D5D5 | 0x9CD24E4E | 0x49E0A9A9 |
| 0xB8 | 0xD8B46C6C | 0xACFA5656 | 0xF307F4F4 | 0xCF25EAEA | 0xCAAF6565 | 0xF48E7A7A | 0x47E9AEAE | 0x10180808 |
| 0xC0 | 0x6FD5BABA | 0xF0887878 | 0x4A6F2525 | 0x5C722E2E | 0x38241C1C | 0x57F1A6A6 | 0x73C7B4B4 | 0x9751C6C6 |
| 0xC8 | 0xCB23E8E8 | 0xA17CDDDD | 0xE89C7474 | 0x3E211F1F | 0x96DD4B4B | 0x61DCBDBD | 0xD868B8B | 0xF858A8A |
| 0xD0 | 0xE0907070 | 0x7C423E3E | 0x71C4B5B5 | 0xCCAA6666 | 0x90D84848 | 0x6050303 | 0xF701F6F6 | 0x1C120E0E |
| 0xD8 | 0xC2A36161 | 0x6A5F3535 | 0xAEF95757 | 0x69D0B9B9 | 0x17918686 | 0x9958C1C1 | 0x3A271D1D | 0x27B99E9E |
| 0xE0 | 0xD938E1E1 | 0xEB13F8F8 | 0x2BB39898 | 0x22331111 | 0xD2BB6969 | 0xA970D9D9 | 0x7898E8E | 0x33A79494 |
| 0xE8 | 0x2DB69B9B | 0x3C221E1E | 0x15928787 | 0xC920E9E9 | 0x8749CECE | 0xAAFF5555 | 0x50782828 | 0xA57ADFDF |
| 0xF0 | 0x38F8C8C | 0x59F8A1A1 | 0x9808989 | 0x1A170D0D | 0x65DABFBF | 0xD731E6E6 | 0x84C64242 | 0xD0B86868 |
| 0xF8 | 0x82C34141 | 0x29B09999 | 0x5A772D2D | 0x1E110F0F | 0x7BCBB0B0 | 0xA8FC5454 | 0x6DD6BBBB | 0x2C3A1616 |

Table 6: S1_T3

S2_T0:

| 0x00 | 0x6f25254a | 0x6c242448 | 0x957373e6 | 0xa96767ce | 0x10d7d7c7 | 0x9baeae35 | 0xe45c5cb8 | 0x50303060 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x08 | 0x85a4a421 | 0x5beeeeb5 | 0xb26e6edc | 0x34cbcbff | 0x877d7dfa | 0xb6b5b503 | 0xef82826d | 0x04dbdbdf |
| 0x10 | 0x45e4e4a1 | 0xfb8e8e75 | 0xd8484890 | 0xdb494992 | 0xd14f4f9e | 0xe75d5dba | 0xbe6a6ad4 | 0x887878f0 |
| 0x18 | 0x907070e0 | 0xf1888879 | 0x51e8e8b9 | 0xe15f5fbe | 0xe25e5ebc | 0xe5848461 | 0xaf6565ca | 0x4fe2e2ad |
| 0x20 | 0x01d8d8d9 | 0x52e9e9bb | 0x3dccccf1 | 0x5eededb3 | 0xc0404080 | 0x712f2f5e | 0x33111122 | 0x78282850 |
| 0x28 | 0xf95757ae | 0x1fd2d2cd | 0x9dacac31 | 0x4ce3e3af | 0xde4a4a94 | 0x3f15152a | 0x2d1b1b36 | 0xa2b9b91b |
| 0x30 | 0xbfb2b20d | 0xe9808069 | 0xe6858563 | 0x83a6a625 | 0x722e2e5c | 0x06020204 | 0xc947478e | 0x7b292952 |
| 0x38 | 0x0907070e | 0xdd4b4b96 | 0x120e0e1c | 0x2ac1c1eb | 0xf35151a2 | 0x97aaaa3d | 0xf289897b | 0x15d4d4c1 |
| 0x40 | 0x37cacafd | 0x03010102 | 0xca46468c | 0xbcb3b30f | 0x58efefb7 | 0x0eddddd3 | 0xcc444488 | 0x8d7b7bf6 |
| 0x48 | 0x2fc2c2ed | 0x817f7ffe | 0xabbebe15 | 0x2cc3c3ef | 0xc89f9f57 | 0x60202040 | 0xd44c4c98 | 0xac6464c8 |
| 0x50 | 0xec83836f | 0x8fa2a22d | 0xb86868d0 | 0xc6424284 | 0x35131326 | 0xb5b4b401 | 0xc3414182 | 0x3ecdcdf3 |
| 0x58 | 0xa7baba1d | 0x23c6c6e5 | 0xa4bbbb1f | 0xb76d6dda | 0xd74d4d9a | 0x937171e2 | 0x63212142 | 0x75f4f481 |
| 0x60 | 0xfe8d8d73 | 0xb9b0b009 | 0x46e5e5a3 | 0xdc93934f | 0x6bfefe95 | 0xf88f8f77 | 0x43e6e6a5 | 0x38cfcff7 |
| 0x68 | 0xc5434386 | 0xcf45458a | 0x53313162 | 0x66222244 | 0x5937376e | 0x5a36366c | 0xd3969645 | 0x67fafa9d |
| 0x70 | 0xadbcbc11 | 0x110f0f1e | 0x18080810 | 0xf65252a4 | 0x271d1d3a | 0xff5555aa | 0x2e1a1a34 | 0x26c5c5e3 |
| 0x78 | 0xd24e4e9c | 0x65232346 | 0xbb6969d2 | 0x8e7a7af4 | 0xdf92924d | 0x68ffff97 | 0xed5b5bb6 | 0xee5a5ab4 |
| 0x80 | 0x54ebebbf | 0xc79a9a5d | 0x241c1c38 | 0x92a9a93b | 0x1ad1d1cb | 0x827e7efc | 0x170d0d1a | 0x6dfcfc91 |
| 0x88 | 0xf05050a0 | 0xf78a8a7d | 0xb3b6b605 | 0xa66262c4 | 0x76f5f583 | 0x1e0a0a14 | 0x61f8f899 | 0x0ddcdcd1 |
| 0x90 | 0x05030306 | 0x443c3c78 | 0x140c0c18 | 0x4b393972 | 0x7af1f18b | 0xa1b8b819 | 0x7cf3f38f | 0x473d3d7a |
| 0x98 | 0x7ff2f28d | 0x16d5d5c3 | 0xd0979747 | 0xaa6666cc | 0xea81816b | 0x56323264 | 0x89a0a029 | 0x00000000 |
| 0xA0 | 0x0a06060c | 0x3bcecef5 | 0x73f6f685 | 0x57eaeabd | 0xb0b7b707 | 0x3917172e | 0x70f7f787 | 0xfd8c8c71 |
| 0xA8 | 0x8b7979f2 | 0x13d6d6c5 | 0x80a7a727 | 0xa8bfbf17 | 0xf48b8b7f | 0x413f3f7e | 0x211f1f3e | 0xf55353a6 |
| 0xB0 | 0xa56363c6 | 0x9f7575ea | 0x5f35356a | 0x742c2c58 | 0xa06060c0 | 0x6efdfd93 | 0x6927274e | 0x1cd3d3cf |
| 0xB8 | 0xd5949441 | 0x86a5a523 | 0x847c7cf8 | 0x8aa1a12b | 0x0f05050a | 0xe85858b0 | 0x772d2d5a | 0xaebdbd13 |
| 0xC0 | 0x02d9d9db | 0x20c7c7e7 | 0x98afaf37 | 0xbd6b6bd6 | 0xfc5454a8 | 0x1d0b0b16 | 0x49e0e0a9 | 0x48383870 |
| 0xC8 | 0x0c040408 | 0x31c8c8f9 | 0xce9d9d53 | 0x40e7e7a7 | 0x3c141428 | 0xbab1b10b | 0xe0878767 | 0xcd9c9c51 |
| 0xD0 | 0x08dfdfd7 | 0xb16f6fde | 0x62f9f99b | 0x07dadadd | 0x7e2a2a54 | 0x25c4c4e1 | 0xeb5959b2 | 0x3a16162c |
| 0xD8 | 0x9c7474e8 | 0xda91914b | 0x94abab3f | 0x6a26264c | 0xa36161c2 | 0x9a7676ec | 0x5c343468 | 0x7d2b2b56 |
| 0xE0 | 0x9eadad33 | 0xc299995b | 0x64fbfb9f | 0x967272e4 | 0x5dececb1 | 0x55333366 | 0x36121224 | 0x0bdeded5 |
| 0xE8 | 0xc1989859 | 0x4d3b3b76 | 0x29c0c0e9 | 0xc49b9b5f | 0x423e3e7c | 0x28181830 | 0x30101020 | 0x4e3a3a74 |
| 0xF0 | 0xfa5656ac | 0x4ae1e1ab | 0x997777ee | 0x32c9c9fb | 0x221e1e3c | 0xcb9e9e55 | 0xd6959543 | 0x8ca3a32f |
| 0xF8 | 0xd9909049 | 0x2b191932 | 0x91a8a839 | 0xb46c6cd8 | 0x1b090912 | 0x19d0d0c9 | 0x79f0f089 | 0xe3868665 |

Table 7: S2_T0

S2_T1:

| 0x00 | 0x25254a6f | 0x2424486c | 0x7373e695 | 0x6767cea9 | 0xd7d7c710 | 0xaeae359b | 0x5c5cb8e4 | 0x30306050 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x08 | 0xa4a42185 | 0xeeeeb55b | 0x6e6edcb2 | 0xcbcbff34 | 0x7d7dfa87 | 0xb5b503b6 | 0x82826def | 0xdbdbdf04 |
| 0x10 | 0xe4e4a145 | 0x8e8e75fb | 0x484890d8 | 0x494992db | 0x4f4f9ed1 | 0x5d5dbae7 | 0x6a6ad4be | 0x7878f088 |
| 0x18 | 0x7070e090 | 0x888879f1 | 0xe8e8b951 | 0x5f5fbee1 | 0x5e5ebce2 | 0x848461e5 | 0x6565caaf | 0xe2e2ad4f |
| 0x20 | 0xd8d8d901 | 0xe9e9bb52 | 0xccccf13d | 0xededb35e | 0x404080c0 | 0x2f2f5e71 | 0x11112233 | 0x28285078 |
| 0x28 | 0x5757aef9 | 0xd2d2cd1f | 0xacac319d | 0xe3e3af4c | 0x4a4a94de | 0x15152a3f | 0x1b1b362d | 0xb9b91ba2 |
| 0x30 | 0xb2b20dbf | 0x808069e9 | 0x858563e6 | 0xa6a62583 | 0x2e2e5c72 | 0x02020406 | 0x47478ec9 | 0x2929527b |
| 0x38 | 0x07070e09 | 0x4b4b96dd | 0x0e0e1c12 | 0xc1c1eb2a | 0x5151a2f3 | 0xaaaa3d97 | 0x89897bf2 | 0xd4d4c115 |
| 0x40 | 0xcacafd37 | 0x01010203 | 0x46468cca | 0xb3b30fbc | 0xefefb758 | 0xddddd30e | 0x444488cc | 0x7b7bf68d |
| 0x48 | 0xc2c2ed2f | 0x7f7ffe81 | 0xbebe15ab | 0xc3c3ef2c | 0x9f9f57c8 | 0x20204060 | 0x4c4c98d4 | 0x6464c8ac |
| 0x50 | 0x83836fec | 0xa2a22d8f | 0x6868d0b8 | 0x424284c6 | 0x13132635 | 0xb4b401b5 | 0x414182c3 | 0xcdcdf33e |
| 0x58 | 0xbaba1da7 | 0xc6c6e523 | 0xbbbb1fa4 | 0x6d6ddab7 | 0x4d4d9ad7 | 0x7171e293 | 0x21214263 | 0xf4f48175 |
| 0x60 | 0x8d8d73fe | 0xb0b009b9 | 0xe5e5a346 | 0x93934fdc | 0xfefe956b | 0x8f8f77f8 | 0xe6e6a543 | 0xcfcff738 |
| 0x68 | 0x434386c5 | 0x45458acf | 0x31316253 | 0x22224466 | 0x37376e59 | 0x36366c5a | 0x969645d3 | 0xfafa9d67 |
| 0x70 | 0xbcbc11ad | 0x0f0f1e11 | 0x08081018 | 0x5252a4f6 | 0x1d1d3a27 | 0x5555aaff | 0x1a1a342e | 0xc5c5e326 |
| 0x78 | 0x4e4e9cd2 | 0x23234665 | 0x6969d2bb | 0x7a7af48e | 0x92924ddf | 0xffff9768 | 0x5b5bb6ed | 0x5a5ab4ee |
| 0x80 | 0xebebbf54 | 0x9a9a5dc7 | 0x1c1c3824 | 0xa9a93b92 | 0xd1d1cb1a | 0x7e7efc82 | 0x0d0d1a17 | 0xfcfc916d |
| 0x88 | 0x5050a0f0 | 0x8a8a7df7 | 0xb6b605b3 | 0x6262c4a6 | 0xf5f58376 | 0x0a0a141e | 0xf8f89961 | 0xdcdcd10d |
| 0x90 | 0x03030605 | 0x3c3c7844 | 0x0c0c1814 | 0x3939724b | 0xf1f18b7a | 0xb8b819a1 | 0xf3f38f7c | 0x3d3d7a47 |
| 0x98 | 0xf2f28d7f | 0xd5d5c316 | 0x979747d0 | 0x6666ccaa | 0x81816bea | 0x32326456 | 0xa0a02989 | 0x00000000 |
| 0xA0 | 0x06060c0a | 0xcecef53b | 0xf6f68573 | 0xeaeabd57 | 0xb7b707b0 | 0x17172e39 | 0xf7f78770 | 0x8c8c71fd |
| 0xA8 | 0x7979f28b | 0xd6d6c513 | 0xa7a72780 | 0xbfbf17a8 | 0x8b8b7ff4 | 0x3f3f7e41 | 0x1f1f3e21 | 0x5353a6f5 |
| 0xB0 | 0x6363c6a5 | 0x7575ea9f | 0x35356a5f | 0x2c2c5874 | 0x6060c0a0 | 0xfdfd936e | 0x27274e69 | 0xd3d3cf1c |
| 0xB8 | 0x949441d5 | 0xa5a52386 | 0x7c7cf884 | 0xa1a12b8a | 0x05050a0f | 0x5858b0e8 | 0x2d2d5a77 | 0xbdbd13ae |
| 0xC0 | 0xd9d9db02 | 0xc7c7e720 | 0xafaf3798 | 0x6b6bd6bd | 0x5454a8fc | 0x0b0b161d | 0xe0e0a949 | 0x38387048 |
| 0xC8 | 0x0404080c | 0xc8c8f931 | 0x9d9d53ce | 0xe7e7a740 | 0x1414283c | 0xb1b10bba | 0x878767e0 | 0x9c9c51cd |
| 0xD0 | 0xdfdfd708 | 0x6f6fdeb1 | 0xf9f99b62 | 0xdadadd07 | 0x2a2a547e | 0xc4c4e125 | 0x5959b2eb | 0x16162c3a |
| 0xD8 | 0x7474e89c | 0x91914bda | 0xabab3f94 | 0x26264c6a | 0x6161c2a3 | 0x7676ec9a | 0x3434685c | 0x2b2b567d |
| 0xE0 | 0xadad339e | 0x99995bc2 | 0xfbfb9f64 | 0x7272e496 | 0xececb15d | 0x33336655 | 0x12122436 | 0xdeded50b |
| 0xE8 | 0x989859c1 | 0x3b3b764d | 0xc0c0e929 | 0x9b9b5fc4 | 0x3e3e7c42 | 0x18183028 | 0x10102030 | 0x3a3a744e |
| 0xF0 | 0x5656acfa | 0xe1e1ab4a | 0x7777ee99 | 0xc9c9fb32 | 0x1e1e3c22 | 0x9e9e55cb | 0x959543d6 | 0xa3a32f8c |
| 0xF8 | 0x909049d9 | 0x1919322b | 0xa8a83991 | 0x6c6cd8b4 | 0x0909121b | 0xd0d0c919 | 0xf0f08979 | 0x868665e3 |

Table 8: S2_T1

S2_T2:

| 0x00 | 0x254a6f25 | 0x24486c24 | 0x73e69573 | 0x67cea967 | 0xd7c710d7 | 0xae359bae | 0x5cb8e45c | 0x30605030 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x08 | 0xa42185a4 | 0xeeb55bee | 0x6edcb26e | 0xcbff34cb | 0x7dfa877d | 0xb503b6b5 | 0x826def82 | 0xdbdf04db |
| 0x10 | 0xe4a145e4 | 0x8e75fb8e | 0x4890d848 | 0x4992db49 | 0x4f9ed14f | 0x5dbae75d | 0x6ad4be6a | 0x78f08878 |
| 0x18 | 0x70e09070 | 0x8879f188 | 0xe8b951e8 | 0x5fbee15f | 0x5ebce25e | 0x8461e584 | 0x65caaf65 | 0xe2ad4fe2 |
| 0x20 | 0xd8d901d8 | 0xe9bb52e9 | 0xccf13dcc | 0xedb35eed | 0x4080c040 | 0x2f5e712f | 0x11223311 | 0x28507828 |
| 0x28 | 0x57aef957 | 0xd2cd1fd2 | 0xac319dac | 0xe3af4ce3 | 0x4a94de4a | 0x152a3f15 | 0x1b362d1b | 0xb91ba2b9 |
| 0x30 | 0xb20dbfb2 | 0x8069e980 | 0x8563e685 | 0xa62583a6 | 0x2e5c722e | 0x02040602 | 0x478ec947 | 0x29527b29 |
| 0x38 | 0x070e0907 | 0x4b96dd4b | 0x0e1c120e | 0xc1eb2ac1 | 0x51a2f351 | 0xaa3d97aa | 0x897bf289 | 0xd4c115d4 |
| 0x40 | 0xcafd37ca | 0x01020301 | 0x468cca46 | 0xb30fbcb3 | 0xefb758ef | 0xddd30edd | 0x4488cc44 | 0x7bf68d7b |
| 0x48 | 0xc2ed2fc2 | 0x7ffe817f | 0xbe15abbe | 0xc3ef2cc3 | 0x9f57c89f | 0x20406020 | 0x4c98d44c | 0x64c8ac64 |
| 0x50 | 0x836fec83 | 0xa22d8fa2 | 0x68d0b868 | 0x4284c642 | 0x13263513 | 0xb401b5b4 | 0x4182c341 | 0xcdf33ecd |
| 0x58 | 0xba1da7ba | 0xc6e523c6 | 0xbb1fa4bb | 0x6ddab76d | 0x4d9ad74d | 0x71e29371 | 0x21426321 | 0xf48175f4 |
| 0x60 | 0x8d73fe8d | 0xb009b9b0 | 0xe5a346e5 | 0x934fdc93 | 0xfe956bfe | 0x8f77f88f | 0xe6a543e6 | 0xcff738cf |
| 0x68 | 0x4386c543 | 0x458acf45 | 0x31625331 | 0x22446622 | 0x376e5937 | 0x366c5a36 | 0x9645d396 | 0xfa9d67fa |
| 0x70 | 0xbc11adbc | 0x0f1e110f | 0x08101808 | 0x52a4f652 | 0x1d3a271d | 0x55aaff55 | 0x1a342e1a | 0xc5e326c5 |
| 0x78 | 0x4e9cd24e | 0x23466523 | 0x69d2bb69 | 0x7af48e7a | 0x924ddf92 | 0xff9768ff | 0x5bb6ed5b | 0x5ab4ee5a |
| 0x80 | 0xebbf54eb | 0x9a5dc79a | 0x1c38241c | 0xa93b92a9 | 0xd1cb1ad1 | 0x7efc827e | 0x0d1a170d | 0xfc916dfc |
| 0x88 | 0x50a0f050 | 0x8a7df78a | 0xb605b3b6 | 0x62c4a662 | 0xf58376f5 | 0x0a141e0a | 0xf89961f8 | 0xdcd10ddc |
| 0x90 | 0x03060503 | 0x3c78443c | 0x0c18140c | 0x39724b39 | 0xf18b7af1 | 0xb819a1b8 | 0xf38f7cf3 | 0x3d7a473d |
| 0x98 | 0xf28d7ff2 | 0xd5c316d5 | 0x9747d097 | 0x66ccaa66 | 0x816bea81 | 0x32645632 | 0xa02989a0 | 0x00000000 |
| 0xA0 | 0x060c0a06 | 0xcef53bce | 0xf68573f6 | 0xeabd57ea | 0xb707b0b7 | 0x172e3917 | 0xf78770f7 | 0x8c71fd8c |
| 0xA8 | 0x79f28b79 | 0xd6c513d6 | 0xa72780a7 | 0xbf17a8bf | 0x8b7ff48b | 0x3f7e413f | 0x1f3e211f | 0x53a6f553 |
| 0xB0 | 0x63c6a563 | 0x75ea9f75 | 0x356a5f35 | 0x2c58742c | 0x60c0a060 | 0xfd936efd | 0x274e6927 | 0xd3cf1cd3 |
| 0xB8 | 0x9441d594 | 0xa52386a5 | 0x7cf8847c | 0xa12b8aa1 | 0x050a0f05 | 0x58b0e858 | 0x2d5a772d | 0xbd13aebd |
| 0xC0 | 0xd9db02d9 | 0xc7e720c7 | 0xaf3798af | 0x6bd6bd6b | 0x54a8fc54 | 0x0b161d0b | 0xe0a949e0 | 0x38704838 |
| 0xC8 | 0x04080c04 | 0xc8f931c8 | 0x9d53ce9d | 0xe7a740e7 | 0x14283c14 | 0xb10bbab1 | 0x8767e087 | 0x9c51cd9c |
| 0xD0 | 0xdfd708df | 0x6fdeb16f | 0xf99b62f9 | 0xdadd07da | 0x2a547e2a | 0xc4e125c4 | 0x59b2eb59 | 0x162c3a16 |
| 0xD8 | 0x74e89c74 | 0x914bda91 | 0xab3f94ab | 0x264c6a26 | 0x61c2a361 | 0x76ec9a76 | 0x34685c34 | 0x2b567d2b |
| 0xE0 | 0xad339ead | 0x995bc299 | 0xfb9f64fb | 0x72e49672 | 0xecb15dec | 0x33665533 | 0x12243612 | 0xded50bde |
| 0xE8 | 0x9859c198 | 0x3b764d3b | 0xc0e929c0 | 0x9b5fc49b | 0x3e7c423e | 0x18302818 | 0x10203010 | 0x3a744e3a |
| 0xF0 | 0x56acfa56 | 0xe1ab4ae1 | 0x77ee9977 | 0xc9fb32c9 | 0x1e3c221e | 0x9e55cb9e | 0x9543d695 | 0xa32f8ca3 |
| 0xF8 | 0x9049d990 | 0x19322b19 | 0xa83991a8 | 0x6cd8b46c | 0x09121b09 | 0xd0c919d0 | 0xf08979f0 | 0x8665e386 |

Table 9: S2_T2

S2_T3:

| 0x00 | 0x4a6f2525 | 0x486c2424 | 0xe6957373 | 0xcea96767 | 0xc710d7d7 | 0x359baeae | 0xb8e45c5c | 0x60503030 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x08 | 0x2185a4a4 | 0xb55beeee | 0xdcb26e6e | 0xff34cbcb | 0xfa877d7d | 0x03b6b5b5 | 0x6def8282 | 0xdf04dbdb |
| 0x10 | 0xa145e4e4 | 0x75fb8e8e | 0x90d84848 | 0x92db4949 | 0x9ed14f4f | 0xbae75d5d | 0xd4be6a6a | 0xf0887878 |
| 0x18 | 0xe0907070 | 0x79f18888 | 0xb951e8e8 | 0xbee15f5f | 0xbce25e5e | 0x61e58484 | 0xcaaf6565 | 0xad4fe2e2 |
| 0x20 | 0xd901d8d8 | 0xbb52e9e9 | 0xf13dcccc | 0xb35eeded | 0x80c04040 | 0x5e712f2f | 0x22331111 | 0x50782828 |
| 0x28 | 0xaef95757 | 0xcd1fd2d2 | 0x319dacac | 0xaf4ce3e3 | 0x94de4a4a | 0x2a3f1515 | 0x362d1b1b | 0x1ba2b9b9 |
| 0x30 | 0x0dbfb2b2 | 0x69e98080 | 0x63e68585 | 0x2583a6a6 | 0x5c722e2e | 0x04060202 | 0x8ec94747 | 0x527b2929 |
| 0x38 | 0x0e090707 | 0x96dd4b4b | 0x1c120e0e | 0xeb2ac1c1 | 0xa2f35151 | 0x3d97aaaa | 0x7bf28989 | 0xc115d4d4 |
| 0x40 | 0xfd37caca | 0x02030101 | 0x8cca4646 | 0x0fbcb3b3 | 0xb758efef | 0xd30edddd | 0x88cc4444 | 0xf68d7b7b |
| 0x48 | 0xed2fc2c2 | 0xfe817f7f | 0x15abbebe | 0xef2cc3c3 | 0x57c89f9f | 0x40602020 | 0x98d44c4c | 0xc8ac6464 |
| 0x50 | 0x6fec8383 | 0x2d8fa2a2 | 0xd0b86868 | 0x84c64242 | 0x26351313 | 0x01b5b4b4 | 0x82c34141 | 0xf33ecdcd |
| 0x58 | 0x1da7baba | 0xe523c6c6 | 0x1fa4bbbb | 0xdab76d6d | 0x9ad74d4d | 0xe2937171 | 0x42632121 | 0x8175f4f4 |
| 0x60 | 0x73fe8d8d | 0x09b9b0b0 | 0xa346e5e5 | 0x4fdc9393 | 0x956bfefe | 0x77f88f8f | 0xa543e6e6 | 0xf738cfcf |
| 0x68 | 0x86c54343 | 0x8acf4545 | 0x62533131 | 0x44662222 | 0x6e593737 | 0x6c5a3636 | 0x45d39696 | 0x9d67fafa |
| 0x70 | 0x11adbcbc | 0x1e110f0f | 0x10180808 | 0xa4f65252 | 0x3a271d1d | 0xaaff5555 | 0x342e1a1a | 0xe326c5c5 |
| 0x78 | 0x9cd24e4e | 0x46652323 | 0xd2bb6969 | 0xf48e7a7a | 0x4ddf9292 | 0x9768ffff | 0xb6ed5b5b | 0xb4ee5a5a |
| 0x80 | 0xbf54ebeb | 0x5dc79a9a | 0x38241c1c | 0x3b92a9a9 | 0xcb1ad1d1 | 0xfc827e7e | 0x1a170d0d | 0x916dfcfc |
| 0x88 | 0xa0f05050 | 0x7df78a8a | 0x05b3b6b6 | 0xc4a66262 | 0x8376f5f5 | 0x141e0a0a | 0x9961f8f8 | 0xd10ddcdc |
| 0x90 | 0x06050303 | 0x78443c3c | 0x18140c0c | 0x724b3939 | 0x8b7af1f1 | 0x19a1b8b8 | 0x8f7cf3f3 | 0x7a473d3d |
| 0x98 | 0x8d7ff2f2 | 0xc316d5d5 | 0x47d09797 | 0xccaa6666 | 0x6bea8181 | 0x64563232 | 0x2989a0a0 | 0x00000000 |
| 0xA0 | 0x0c0a0606 | 0xf53bcece | 0x8573f6f6 | 0xbd57eaea | 0x07b0b7b7 | 0x2e391717 | 0x8770f7f7 | 0x71fd8c8c |
| 0xA8 | 0xf28b7979 | 0xc513d6d6 | 0x2780a7a7 | 0x17a8bfbf | 0x7ff48b8b | 0x7e413f3f | 0x3e211f1f | 0xa6f55353 |
| 0xB0 | 0xc6a56363 | 0xea9f7575 | 0x6a5f3535 | 0x58742c2c | 0xc0a06060 | 0x936efdfd | 0x4e692727 | 0xcf1cd3d3 |
| 0xB8 | 0x41d59494 | 0x2386a5a5 | 0xf8847c7c | 0x2b8aa1a1 | 0x0a0f0505 | 0xb0e85858 | 0x5a772d2d | 0x13aebdbd |
| 0xC0 | 0xdb02d9d9 | 0xe720c7c7 | 0x3798afaf | 0xd6bd6b6b | 0xa8fc5454 | 0x161d0b0b | 0xa949e0e0 | 0x70483838 |
| 0xC8 | 0x080c0404 | 0xf931c8c8 | 0x53ce9d9d | 0xa740e7e7 | 0x283c1414 | 0x0bbab1b1 | 0x67e08787 | 0x51cd9c9c |
| 0xD0 | 0xd708dfdf | 0xdeb16f6f | 0x9b62f9f9 | 0xdd07dada | 0x547e2a2a | 0xe125c4c4 | 0xb2eb5959 | 0x2c3a1616 |
| 0xD8 | 0xe89c7474 | 0x4bda9191 | 0x3f94abab | 0x4c6a2626 | 0xc2a36161 | 0xec9a7676 | 0x685c3434 | 0x567d2b2b |
| 0xE0 | 0x339eadad | 0x5bc29999 | 0x9f64fbfb | 0xe4967272 | 0xb15decec | 0x66553333 | 0x24361212 | 0xd50bdede |
| 0xE8 | 0x59c19898 | 0x764d3b3b | 0xe929c0c0 | 0x5fc49b9b | 0x7c423e3e | 0x30281818 | 0x20301010 | 0x744e3a3a |
| 0xF0 | 0xacfa5656 | 0xab4ae1e1 | 0xee997777 | 0xfb32c9c9 | 0x3c221e1e | 0x55cb9e9e | 0x43d69595 | 0x2f8ca3a3 |
| 0xF8 | 0x49d99090 | 0x322b1919 | 0x3991a8a8 | 0xd8b46c6c | 0x121b0909 | 0xc919d0d0 | 0x8979f0f0 | 0x65e38686 |

Table 10: S2_T3

## Definitions of tables for the LFSR

MULalpha is defined by the following table:

| 0x00 | 0x00000000 | 0xE19FCF13 | 0x6B973726 | 0x8A08F835 | 0xD6876E4C | 0x3718A15F | 0xBD10596A | 0x5C8F9679 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x08 | 0x05A7DC98 | 0xE438138B | 0x6E30EBBE | 0x8FAF24AD | 0xD320B2D4 | 0x32BF7DC7 | 0xB8B785F2 | 0x59284AE1 |
| 0x10 | 0x0AE71199 | 0xEB78DE8A | 0x617026BF | 0x80EFE9AC | 0xDC607FD5 | 0x3DFFB0C6 | 0xB7F748F3 | 0x566887E0 |
| 0x18 | 0x0F40CD01 | 0xEEDF0212 | 0x64D7FA27 | 0x85483534 | 0xD9C7A34D | 0x38586C5E | 0xB250946B | 0x53CF5B78 |
| 0x20 | 0x1467229B | 0xF5F8ED88 | 0x7FF015BD | 0x9E6FDAAE | 0xC2E04CD7 | 0x237F83C4 | 0xA9777BF1 | 0x48E8B4E2 |
| 0x28 | 0x11C0FE03 | 0xF05F3110 | 0x7A57C925 | 0x9BC80636 | 0xC747904F | 0x26D85F5C | 0xACD0A769 | 0x4D4F687A |
| 0x30 | 0x1E803302 | 0xFF1FFC11 | 0x75170424 | 0x9488CB37 | 0xC8075D4E | 0x2998925D | 0xA3906A68 | 0x420FA57B |
| 0x38 | 0x1B27EF9A | 0xFAB82089 | 0x70B0D8BC | 0x912F17AF | 0xCDA081D6 | 0x2C3F4EC5 | 0xA637B6F0 | 0x47A879E3 |
| 0x40 | 0x28CE449F | 0xC9518B8C | 0x435973B9 | 0xA2C6BCAA | 0xFE492AD3 | 0x1FD6E5C0 | 0x95DE1DF5 | 0x7441D2E6 |
| 0x48 | 0x2D699807 | 0xCCF65714 | 0x46FEAF21 | 0xA7616032 | 0xFBEEF64B | 0x1A713958 | 0x9079C16D | 0x71E60E7E |
| 0x50 | 0x22295506 | 0xC3B69A15 | 0x49BE6220 | 0xA821AD33 | 0xF4AE3B4A | 0x1531F459 | 0x9F390C6C | 0x7EA6C37F |
| 0x58 | 0x278E899E | 0xC611468D | 0x4C19BEB8 | 0xAD8671AB | 0xF109E7D2 | 0x109628C1 | 0x9A9ED0F4 | 0x7B011FE7 |
| 0x60 | 0x3CA96604 | 0xDD36A917 | 0x573E5122 | 0xB6A19E31 | 0xEA2E0848 | 0x0BB1C75B | 0x81B93F6E | 0x6026F07D |
| 0x68 | 0x390EBA9C | 0xD891758F | 0x52998DBA | 0xB30642A9 | 0xEF89D4D0 | 0x0E161BC3 | 0x841EE3F6 | 0x65812CE5 |
| 0x70 | 0x364E779D | 0xD7D1B88E | 0x5DD940BB | 0xBC468FA8 | 0xE0C919D1 | 0x0156D6C2 | 0x8B5E2EF7 | 0x6AC1E1E4 |
| 0x78 | 0x33E9AB05 | 0xD2766416 | 0x587E9C23 | 0xB9E15330 | 0xE56EC549 | 0x04F10A5A | 0x8EF9F26F | 0x6F663D7C |
| 0x80 | 0x50358897 | 0xB1AA4784 | 0x3BA2BFB1 | 0xDA3D70A2 | 0x86B2E6DB | 0x672D29C8 | 0xED25D1FD | 0x0CBA1EEE |
| 0x88 | 0x5592540F | 0xB40D9B1C | 0x3E056329 | 0xDF9AAC3A | 0x83153A43 | 0x628AF550 | 0xE8820D65 | 0x091DC276 |
| 0x90 | 0x5AD2990E | 0xBB4D561D | 0x3145AE28 | 0xD0DA613B | 0x8C55F742 | 0x6DCA3851 | 0xE7C2C064 | 0x065D0F77 |
| 0x98 | 0x5F754596 | 0xBEEA8A85 | 0x34E272B0 | 0xD57DBDA3 | 0x89F22BDA | 0x686DE4C9 | 0xE2651CFC | 0x03FAD3EF |
| 0xA0 | 0x4452AA0C | 0xA5CD651F | 0x2FC59D2A | 0xCE5A5239 | 0x92D5C440 | 0x734A0B53 | 0xF942F366 | 0x18DD3C75 |
| 0xA8 | 0x41F57694 | 0xA06AB987 | 0x2A6241B2 | 0xCBFD8EA1 | 0x977218D8 | 0x76EDD7CB | 0xFCE52FFE | 0x1D7AE0ED |
| 0xB0 | 0x4EB5BB95 | 0xAF2A7486 | 0x25228CB3 | 0xC4BD43A0 | 0x9832D5D9 | 0x79AD1ACA | 0xF3A5E2FF | 0x123A2DEC |
| 0xB8 | 0x4B12670D | 0xAA8DA81E | 0x2085502B | 0xC11A9F38 | 0x9D950941 | 0x7C0AC652 | 0xF6023E67 | 0x179DF174 |
| 0xC0 | 0x78FBCC08 | 0x9964031B | 0x136CFB2E | 0xF2F3343D | 0xAE7CA244 | 0x4FE36D57 | 0xC5EB9562 | 0x24745A71 |
| 0xC8 | 0x7D5C1090 | 0x9CC3DF83 | 0x16CB27B6 | 0xF754E8A5 | 0xABDB7EDC | 0x4A44B1CF | 0xC04C49FA | 0x21D386E9 |
| 0xD0 | 0x721CDD91 | 0x93831282 | 0x198BEAB7 | 0xF81425A4 | 0xA49BB3DD | 0x45047CCE | 0xCF0C84FB | 0x2E934BE8 |
| 0xD8 | 0x77BB0109 | 0x9624CE1A | 0x1C2C362F | 0xFDB3F93C | 0xA13C6F45 | 0x40A3A056 | 0xCAAB5863 | 0x2B349770 |
| 0xE0 | 0x6C9CEE93 | 0x8D032180 | 0x070BD9B5 | 0xE69416A6 | 0xBA1B80DF | 0x5B844FCC | 0xD18CB7F9 | 0x301378EA |
| 0xE8 | 0x693B320B | 0x88A4FD18 | 0x02AC052D | 0xE333CA3E | 0xBFBC5C47 | 0x5E239354 | 0xD42B6B61 | 0x35B4A472 |
| 0xF0 | 0x667BFF0A | 0x87E43019 | 0x0DECC82C | 0xEC73073F | 0xB0FC9146 | 0x51635E55 | 0xDB6BA660 | 0x3AF46973 |
| 0xF8 | 0x63DC2392 | 0x8243EC81 | 0x084B14B4 | 0xE9D4DBA7 | 0xB55B4DDE | 0x54C482CD | 0xDECC7AF8 | 0x3F53B5EB |

Table 11: MULalpha

Example: MULalpha(0x2A) = 0x7A57C925.

DIValpha is defined by the following table:

| 0x00 | 0x00000000 | 0x180F40CD | 0x301E8033 | 0x2811C0FE | 0x603CA966 | 0x7833E9AB | 0x50222955 | 0x482D6998 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x08 | 0xC078FBCC | 0xD877BB01 | 0xF0667BFF | 0xE8693B32 | 0xA04452AA | 0xB84B1267 | 0x905AD299 | 0x88559254 |
| 0x10 | 0x29F05F31 | 0x31FF1FFC | 0x19EEDF02 | 0x01E19FCF | 0x49CCF657 | 0x51C3B69A | 0x79D27664 | 0x61DD36A9 |
| 0x18 | 0xE988A4FD | 0xF187E430 | 0xD99624CE | 0xC1996403 | 0x89B40D9B | 0x91BB4D56 | 0xB9AA8DA8 | 0xA1A5CD65 |
| 0x20 | 0x5249BE62 | 0x4A46FEAF | 0x62573E51 | 0x7A587E9C | 0x32751704 | 0x2A7A57C9 | 0x026B9737 | 0x1A64D7FA |
| 0x28 | 0x923145AE | 0x8A3E0563 | 0xA22FC59D | 0xBA208550 | 0xF20DECC8 | 0xEA02AC05 | 0xC2136CFB | 0xDA1C2C36 |
| 0x30 | 0x7BB9E153 | 0x63B6A19E | 0x4BA76160 | 0x53A821AD | 0x1B854835 | 0x038A08F8 | 0x2B9BC806 | 0x339488CB |
| 0x38 | 0xBBC11A9F | 0xA3CE5A52 | 0x8BDF9AAC | 0x93D0DA61 | 0xDBFDB3F9 | 0xC3F2F334 | 0xEBE333CA | 0xF3EC7307 |
| 0x40 | 0xA492D5C4 | 0xBC9D9509 | 0x948C55F7 | 0x8C83153A | 0xC4AE7CA2 | 0xDCA13C6F | 0xF4B0FC91 | 0xECBFBC5C |
| 0x48 | 0x64EA2E08 | 0x7CE56EC5 | 0x54F4AE3B | 0x4CFBEEF6 | 0x04D6876E | 0x1CD9C7A3 | 0x34C8075D | 0x2CC74790 |
| 0x50 | 0x8D628AF5 | 0x956DCA38 | 0xBD7C0AC6 | 0xA5734A0B | 0xED5E2393 | 0xF551635E | 0xDD40A3A0 | 0xC54FE36D |
| 0x58 | 0x4D1A7139 | 0x551531F4 | 0x7D04F10A | 0x650BB1C7 | 0x2D26D85F | 0x35299892 | 0x1D38586C | 0x053718A1 |
| 0x60 | 0xF6DB6BA6 | 0xEED42B6B | 0xC6C5EB95 | 0xDECAAB58 | 0x96E7C2C0 | 0x8EE8820D | 0xA6F942F3 | 0xBEF6023E |
| 0x68 | 0x36A3906A | 0x2EACD0A7 | 0x06BD1059 | 0x1EB25094 | 0x569F390C | 0x4E9079C1 | 0x6681B93F | 0x7E8EF9F2 |
| 0x70 | 0xDF2B3497 | 0xC724745A | 0xEF35B4A4 | 0xF73AF469 | 0xBF179DF1 | 0xA718DD3C | 0x8F091DC2 | 0x97065D0F |
| 0x78 | 0x1F53CF5B | 0x075C8F96 | 0x2F4D4F68 | 0x37420FA5 | 0x7F6F663D | 0x676026F0 | 0x4F71E60E | 0x577EA6C3 |
| 0x80 | 0xE18D0321 | 0xF98243EC | 0xD1938312 | 0xC99CC3DF | 0x81B1AA47 | 0x99BEEA8A | 0xB1AF2A74 | 0xA9A06AB9 |
| 0x88 | 0x21F5F8ED | 0x39FAB820 | 0x11EB78DE | 0x09E43813 | 0x41C9518B | 0x59C61146 | 0x71D7D1B8 | 0x69D89175 |
| 0x90 | 0xC87D5C10 | 0xD0721CDD | 0xF863DC23 | 0xE06C9CEE | 0xA841F576 | 0xB04EB5BB | 0x985F7545 | 0x80503588 |
| 0x98 | 0x0805A7DC | 0x100AE711 | 0x381B27EF | 0x20146722 | 0x68390EBA | 0x70364E77 | 0x58278E89 | 0x4028CE44 |
| 0xA0 | 0xB3C4BD43 | 0xABCBFD8E | 0x83DA3D70 | 0x9BD57DBD | 0xD3F81425 | 0xCBF754E8 | 0xE3E69416 | 0xFBE9D4DB |
| 0xA8 | 0x73BC468F | 0x6BB30642 | 0x43A2C6BC | 0x5BAD8671 | 0x1380EFE9 | 0x0B8FAF24 | 0x239E6FDA | 0x3B912F17 |
| 0xB0 | 0x9A34E272 | 0x823BA2BF | 0xAA2A6241 | 0xB225228C | 0xFA084B14 | 0xE2070BD9 | 0xCA16CB27 | 0xD2198BEA |
| 0xB8 | 0x5A4C19BE | 0x42435973 | 0x6A52998D | 0x725DD940 | 0x3A70B0D8 | 0x227FF015 | 0x0A6E30EB | 0x12617026 |
| 0xC0 | 0x451FD6E5 | 0x5D109628 | 0x750156D6 | 0x6D0E161B | 0x25237F83 | 0x3D2C3F4E | 0x153DFFB0 | 0x0D32BF7D |
| 0xC8 | 0x85672D29 | 0x9D686DE4 | 0xB579AD1A | 0xAD76EDD7 | 0xE55B844F | 0xFD54C482 | 0xD545047C | 0xCD4A44B1 |
| 0xD0 | 0x6CEF89D4 | 0x74E0C919 | 0x5CF109E7 | 0x44FE492A | 0x0CD320B2 | 0x14DC607F | 0x3CCDA081 | 0x24C2E04C |
| 0xD8 | 0xAC977218 | 0xB49832D5 | 0x9C89F22B | 0x8486B2E6 | 0xCCABDB7E | 0xD4A49BB3 | 0xFCB55B4D | 0xE4BA1B80 |
| 0xE0 | 0x17566887 | 0x0F59284A | 0x2748E8B4 | 0x3F47A879 | 0x776AC1E1 | 0x6F65812C | 0x477441D2 | 0x5F7B011F |
| 0xE8 | 0xD72E934B | 0xCF21D386 | 0xE7301378 | 0xFF3F53B5 | 0xB7123A2D | 0xAF1D7AE0 | 0x870CBA1E | 0x9F03FAD3 |
| 0xF0 | 0x3EA637B6 | 0x26A9777B | 0x0EB8B785 | 0x16B7F748 | 0x5E9A9ED0 | 0x4695DE1D | 0x6E841EE3 | 0x768B5E2E |
| 0xF8 | 0xFEDECC7A | 0xE6D18CB7 | 0xCEC04C49 | 0xD6CF0C84 | 0x9EE2651C | 0x86ED25D1 | 0xAEFCE52F | 0xB6F3A5E2 |

Table 12: DIValpha

# Figures of the SNOW 3G Algorithm

## SNOW 3G Algorithm during key initialisation

This illustrates the operation of SNOW 3G described in chapter 3.4.4 of the normative part.

s11

s5

s2

s1

s0

s15

-1



⊞

R1

R3

R2

⊞

S1

S2

FSM

## SNOW 3G Algorithm during keystream-generation

This illustrates the operation of SNOW 3G described in chapter 3.4.5 of the normative part.

s11

s5

s2

s1

s0

s15

-1



⊞

R1

R3

R2

⊞

zt

S2

S1

FSM

# Simulation Program Listing

## Header file

/*---------------------------------------------------------

*					SNOW_3G.h

*---------------------------------------------------------*/

typedef unsigned char u8;

typedef unsigned int u32;

typedef unsigned long long u64;

/* Initialization.

* Input k[4]: Four 32-bit words making up 128-bit key.

* Input IV[4]: Four 32-bit words making 128-bit initialization variable.

* Output: All the LFSRs and FSM are initialized for key generation.

* See Section 4.1.

*/

void Initialize(u32 k[4], u32 IV[4]);

/* Generation of Keystream.

* input n: number of 32-bit words of keystream.

* input z: space for the generated keystream, assumes

*          memory is allocated already.

* output: generated keystream which is filled in z

* See section 4.2.

*/

void GenerateKeystream(u32 n, u32 *z);

## Code

/*------------------------------------------------------------------------

*				SNOW_3G.c

*------------------------------------------------------------------------*/

#include "SNOW_3G.h"

/* LFSR */

u32 LFSR_S0 = 0x00;

u32 LFSR_S1 = 0x00;

u32 LFSR_S2 = 0x00;

u32 LFSR_S3 = 0x00;

u32 LFSR_S4 = 0x00;

u32 LFSR_S5 = 0x00;

u32 LFSR_S6 = 0x00;

u32 LFSR_S7 = 0x00;

u32 LFSR_S8 = 0x00;

u32 LFSR_S9 = 0x00;

u32 LFSR_S10 = 0x00;

u32 LFSR_S11 = 0x00;

u32 LFSR_S12 = 0x00;

u32 LFSR_S13 = 0x00;

u32 LFSR_S14 = 0x00;

u32 LFSR_S15 = 0x00;

/* FSM */

u32 FSM_R1 = 0x00;

u32 FSM_R2 = 0x00;

u32 FSM_R3 = 0x00;

/* Rijndael S-box SR */

u8 SR[256] = {

0x63,0x7C,0x77,0x7B,0xF2,0x6B,0x6F,0xC5,0x30,0x01,0x67,0x2B,0xFE,0xD7,0xAB,0x76,

0xCA,0x82,0xC9,0x7D,0xFA,0x59,0x47,0xF0,0xAD,0xD4,0xA2,0xAF,0x9C,0xA4,0x72,0xC0,

0xB7,0xFD,0x93,0x26,0x36,0x3F,0xF7,0xCC,0x34,0xA5,0xE5,0xF1,0x71,0xD8,0x31,0x15,

0x04,0xC7,0x23,0xC3,0x18,0x96,0x05,0x9A,0x07,0x12,0x80,0xE2,0xEB,0x27,0xB2,0x75,

0x09,0x83,0x2C,0x1A,0x1B,0x6E,0x5A,0xA0,0x52,0x3B,0xD6,0xB3,0x29,0xE3,0x2F,0x84,

0x53,0xD1,0x00,0xED,0x20,0xFC,0xB1,0x5B,0x6A,0xCB,0xBE,0x39,0x4A,0x4C,0x58,0xCF,

0xD0,0xEF,0xAA,0xFB,0x43,0x4D,0x33,0x85,0x45,0xF9,0x02,0x7F,0x50,0x3C,0x9F,0xA8,

0x51,0xA3,0x40,0x8F,0x92,0x9D,0x38,0xF5,0xBC,0xB6,0xDA,0x21,0x10,0xFF,0xF3,0xD2,

0xCD,0x0C,0x13,0xEC,0x5F,0x97,0x44,0x17,0xC4,0xA7,0x7E,0x3D,0x64,0x5D,0x19,0x73,

0x60,0x81,0x4F,0xDC,0x22,0x2A,0x90,0x88,0x46,0xEE,0xB8,0x14,0xDE,0x5E,0x0B,0xDB,

0xE0,0x32,0x3A,0x0A,0x49,0x06,0x24,0x5C,0xC2,0xD3,0xAC,0x62,0x91,0x95,0xE4,0x79,

0xE7,0xC8,0x37,0x6D,0x8D,0xD5,0x4E,0xA9,0x6C,0x56,0xF4,0xEA,0x65,0x7A,0xAE,0x08,

0xBA,0x78,0x25,0x2E,0x1C,0xA6,0xB4,0xC6,0xE8,0xDD,0x74,0x1F,0x4B,0xBD,0x8B,0x8A,

0x70,0x3E,0xB5,0x66,0x48,0x03,0xF6,0x0E,0x61,0x35,0x57,0xB9,0x86,0xC1,0x1D,0x9E,

0xE1,0xF8,0x98,0x11,0x69,0xD9,0x8E,0x94,0x9B,0x1E,0x87,0xE9,0xCE,0x55,0x28,0xDF,

0x8C,0xA1,0x89,0x0D,0xBF,0xE6,0x42,0x68,0x41,0x99,0x2D,0x0F,0xB0,0x54,0xBB,0x16 };

/* S-box SQ */

u8 SQ[256] = {

0x25,0x24,0x73,0x67,0xD7,0xAE,0x5C,0x30,0xA4,0xEE,0x6E,0xCB,0x7D,0xB5,0x82,0xDB,

0xE4,0x8E,0x48,0x49,0x4F,0x5D,0x6A,0x78,0x70,0x88,0xE8,0x5F,0x5E,0x84,0x65,0xE2,

0xD8,0xE9,0xCC,0xED,0x40,0x2F,0x11,0x28,0x57,0xD2,0xAC,0xE3,0x4A,0x15,0x1B,0xB9,

0xB2,0x80,0x85,0xA6,0x2E,0x02,0x47,0x29,0x07,0x4B,0x0E,0xC1,0x51,0xAA,0x89,0xD4,

0xCA,0x01,0x46,0xB3,0xEF,0xDD,0x44,0x7B,0xC2,0x7F,0xBE,0xC3,0x9F,0x20,0x4C,0x64,

0x83,0xA2,0x68,0x42,0x13,0xB4,0x41,0xCD,0xBA,0xC6,0xBB,0x6D,0x4D,0x71,0x21,0xF4,

0x8D,0xB0,0xE5,0x93,0xFE,0x8F,0xE6,0xCF,0x43,0x45,0x31,0x22,0x37,0x36,0x96,0xFA,

0xBC,0x0F,0x08,0x52,0x1D,0x55,0x1A,0xC5,0x4E,0x23,0x69,0x7A,0x92,0xFF,0x5B,0x5A,

0xEB,0x9A,0x1C,0xA9,0xD1,0x7E,0x0D,0xFC,0x50,0x8A,0xB6,0x62,0xF5,0x0A,0xF8,0xDC,

0x03,0x3C,0x0C,0x39,0xF1,0xB8,0xF3,0x3D,0xF2,0xD5,0x97,0x66,0x81,0x32,0xA0,0x00,

0x06,0xCE,0xF6,0xEA,0xB7,0x17,0xF7,0x8C,0x79,0xD6,0xA7,0xBF,0x8B,0x3F,0x1F,0x53,

0x63,0x75,0x35,0x2C,0x60,0xFD,0x27,0xD3,0x94,0xA5,0x7C,0xA1,0x05,0x58,0x2D,0xBD,

0xD9,0xC7,0xAF,0x6B,0x54,0x0B,0xE0,0x38,0x04,0xC8,0x9D,0xE7,0x14,0xB1,0x87,0x9C,

0xDF,0x6F,0xF9,0xDA,0x2A,0xC4,0x59,0x16,0x74,0x91,0xAB,0x26,0x61,0x76,0x34,0x2B,

0xAD,0x99,0xFB,0x72,0xEC,0x33,0x12,0xDE,0x98,0x3B,0xC0,0x9B,0x3E,0x18,0x10,0x3A,

0x56,0xE1,0x77,0xC9,0x1E,0x9E,0x95,0xA3,0x90,0x19,0xA8,0x6C,0x09,0xD0,0xF0,0x86 };

/* MULx.

* Input V: an 8-bit input.

* Input c: an 8-bit input.

* Output : an 8-bit output.

* See section 3.1.1 for details.

*/

u8 MULx(u8 V, u8 c)

{

if ( V & 0x80 )

return ( (V << 1) ^ c);

else

return ( V << 1);

}

/* MULxPOW.

* Input V: an 8-bit input.

* Input i: a positive integer.

* Input c: an 8-bit input.

* Output : an 8-bit output.

* See section 3.1.2 for details.

*/

u8 MULxPOW(u8 V, u8 i, u8 c)

{

if ( i == 0)

return V;

else

return MULx( MULxPOW( V, i-1, c ), c);

}

/* The function MUL alpha.

* Input c: 8-bit input.

* Output : 32-bit output.

* See section 3.4.2 for details.

*/

u32 MULalpha(u8 c)

{

return ( ( ((u32)MULxPOW(c,  23, 0xa9)) << 24 ) |

( ((u32)MULxPOW(c, 245, 0xa9)) << 16 ) |

( ((u32)MULxPOW(c,  48, 0xa9)) <<  8 ) |

( ((u32)MULxPOW(c, 239, 0xa9))       )    ) ;

}

/* The function DIV alpha.

* Input c: 8-bit input.

* Output : 32-bit output.

* See section 3.4.3 for details.

*/

u32 DIValpha(u8 c)

{

return ( ( ((u32)MULxPOW(c,  16, 0xa9)) << 24 ) |

( ((u32)MULxPOW(c,  39, 0xa9)) << 16 ) |

( ((u32)MULxPOW(c,   6, 0xa9)) <<  8 ) |

( ((u32)MULxPOW(c,  64, 0xa9))       )    ) ;

}

/* The 32x32-bit S-Box S1

* Input: a 32-bit input.

* Output: a 32-bit output of S1 box.

* See section 3.3.1.

*/

u32 S1(u32 w)

{

u8 r0=0, r1=0, r2=0, r3=0;

u8 srw0 = SR[ (u8)((w >> 24) & 0xff) ];

u8 srw1 = SR[ (u8)((w >> 16) & 0xff) ];

u8 srw2 = SR[ (u8)((w >>  8) & 0xff) ];

u8 srw3 = SR[ (u8)((w)       & 0xff) ];

r0 = ( ( MULx( srw0 , 0x1b) )  ^

( srw1 ) ^

( srw2 ) ^

( (MULx( srw3, 0x1b)) ^ srw3 )

);

r1 = ( ( ( MULx( srw0 , 0x1b) ) ^ srw0 ) ^

( MULx(srw1, 0x1b) ) ^

( srw2 ) ^

( srw3 )

);

r2 = ( ( srw0 ) ^

( ( MULx( srw1 , 0x1b) ) ^ srw1 ) ^

( MULx(srw2, 0x1b) ) ^

( srw3 )

);

r3 = ( ( srw0 ) ^

( srw1 ) ^

( ( MULx( srw2 , 0x1b) ) ^ srw2 ) ^

( MULx( srw3, 0x1b) )

);

return ( ( ((u32)r0) << 24 ) | ( ((u32)r1) << 16 ) | ( ((u32)r2) << 8 ) | ( ((u32)r3) ) );

}

/* The 32x32-bit S-Box S2

* Input: a 32-bit input.

* Output: a 32-bit output of S2 box.

* See section 3.3.2.

*/

u32 S2(u32 w)

{

u8 r0=0, r1=0, r2=0, r3=0;

u8 sqw0 = SQ[ (u8)((w >> 24) & 0xff) ];

u8 sqw1 = SQ[ (u8)((w >> 16) & 0xff) ];

u8 sqw2 = SQ[ (u8)((w >>  8) & 0xff) ];

u8 sqw3 = SQ[ (u8)((w)       & 0xff) ];

r0 = ( ( MULx( sqw0 , 0x69) )  ^

( sqw1 ) ^

( sqw2 ) ^

( (MULx( sqw3, 0x69)) ^ sqw3 )

);

r1 = ( ( ( MULx( sqw0 , 0x69) ) ^ sqw0 ) ^

( MULx(sqw1, 0x69) ) ^

( sqw2 ) ^

( sqw3 )

);

r2 = ( ( sqw0 ) ^

( ( MULx( sqw1 , 0x69) ) ^ sqw1 ) ^

( MULx(sqw2, 0x69) ) ^

( sqw3 )

);

r3 = ( ( sqw0 ) ^

( sqw1 ) ^

( ( MULx( sqw2 , 0x69) ) ^ sqw2 ) ^

( MULx( sqw3, 0x69) )

);

return ( ( ((u32)r0) << 24 ) | ( ((u32)r1) << 16 ) | ( ((u32)r2) << 8 ) | ( ((u32)r3) ) );

}

/* Clocking LFSR in initialization mode.

* LFSR Registers S0 to S15 are updated as the LFSR receives a single clock.

* Input F: a 32-bit word comes from output of FSM.

* See section 3.4.4.

*/

void ClockLFSRInitializationMode(u32 F)

{

u32 v = ( ( (LFSR_S0 << 8) & 0xffffff00 )         ^

( MULalpha( (u8)((LFSR_S0>>24) & 0xff) ) )   ^

( LFSR_S2 )                      ^

( (LFSR_S11 >> 8)  & 0x00ffffff )        ^

( DIValpha( (u8)( ( LFSR_S11) & 0xff ) ) ) ^

( F )

);

LFSR_S0 = LFSR_S1;

LFSR_S1 = LFSR_S2;

LFSR_S2 = LFSR_S3;

LFSR_S3 = LFSR_S4;

LFSR_S4 = LFSR_S5;

LFSR_S5 = LFSR_S6;

LFSR_S6 = LFSR_S7;

LFSR_S7 = LFSR_S8;

LFSR_S8 = LFSR_S9;

LFSR_S9 = LFSR_S10;

LFSR_S10 = LFSR_S11;

LFSR_S11 = LFSR_S12;

LFSR_S12 = LFSR_S13;

LFSR_S13 = LFSR_S14;

LFSR_S14 = LFSR_S15;

LFSR_S15 = v;

}

/* Clocking LFSR in keystream mode.

* LFSR Registers S0 to S15 are updated as the LFSR receives a single clock.

* See section 3.4.5.

*/

void ClockLFSRKeyStreamMode()

{

u32 v = ( ( (LFSR_S0 << 8) & 0xffffff00 )         ^

( MULalpha( (u8)((LFSR_S0>>24) & 0xff) ) )   ^

( LFSR_S2 )                      ^

( (LFSR_S11 >> 8)  & 0x00ffffff )        ^

( DIValpha( (u8)( ( LFSR_S11) & 0xff ) ) )

);

LFSR_S0 = LFSR_S1;

LFSR_S1 = LFSR_S2;

LFSR_S2 = LFSR_S3;

LFSR_S3 = LFSR_S4;

LFSR_S4 = LFSR_S5;

LFSR_S5 = LFSR_S6;

LFSR_S6 = LFSR_S7;

LFSR_S7 = LFSR_S8;

LFSR_S8 = LFSR_S9;

LFSR_S9 = LFSR_S10;

LFSR_S10 = LFSR_S11;

LFSR_S11 = LFSR_S12;

LFSR_S12 = LFSR_S13;

LFSR_S13 = LFSR_S14;

LFSR_S14 = LFSR_S15;

LFSR_S15 = v;

}

/* Clocking FSM.

* Produces a 32-bit word F.

* Updates FSM registers R1, R2, R3.

* See Section 3.4.6.

*/

u32 ClockFSM()

{

u32 F = ( ( LFSR_S15 + FSM_R1 ) & 0xffffffff ) ^ FSM_R2 ;

u32 r = ( FSM_R2 + ( FSM_R3 ^ LFSR_S5 ) ) & 0xffffffff ;

FSM_R3 = S2(FSM_R2);

FSM_R2 = S1(FSM_R1);

FSM_R1 = r;

return F;

}

/* Initialization.

* Input k[4]: Four 32-bit words making up 128-bit key.

* Input IV[4]: Four 32-bit words making 128-bit initialization variable.

* Output: All the LFSRs and FSM are initialized for key generation.

* See Section 4.1.

*/

void Initialize(u32 k[4], u32 IV[4])

{

u8 i=0;

u32 F = 0x0;

LFSR_S15 = k[3] ^ IV[0];

LFSR_S14 = k[2];

LFSR_S13 = k[1];

LFSR_S12 = k[0] ^ IV[1];

LFSR_S11 = k[3] ^ 0xffffffff;

LFSR_S10 = k[2] ^ 0xffffffff ^ IV[2];

LFSR_S9  = k[1] ^ 0xffffffff ^ IV[3];

LFSR_S8  = k[0] ^ 0xffffffff;

LFSR_S7 = k[3];

LFSR_S6 = k[2];

LFSR_S5 = k[1];

LFSR_S4 = k[0];

LFSR_S3 = k[3] ^ 0xffffffff;

LFSR_S2 = k[2] ^ 0xffffffff;

LFSR_S1 = k[1] ^ 0xffffffff;

LFSR_S0 = k[0] ^ 0xffffffff;

FSM_R1 = 0x0;

FSM_R2 = 0x0;

FSM_R3 = 0x0;

for(i=0;i<32;i++)

{

F = ClockFSM();

ClockLFSRInitializationMode(F);

}

}

/* Generation of Keystream.

* input n: number of 32-bit words of keystream.

* input z: space for the generated keystream, assumes

*          memory is allocated already.

* output: generated keystream which is filled in z

* See section 4.2.

*/

void GenerateKeystream(u32 n, u32 *ks)

{

u32 t = 0;

u32 F = 0x0;

ClockFSM(); /* Clock FSM once. Discard the output. */

ClockLFSRKeyStreamMode(); /* Clock LFSR in keystream mode once. */

for ( t=0; t<n; t++)

{

F = ClockFSM();             /* STEP 1 */

ks[t] = F ^ LFSR_S0;         /* STEP 2 */

/* Note that ks[t] corresponds to z_{t+1} in section 4.2 */

ClockLFSRKeyStreamMode();   /* STEP 3 */

}

}

/*------------------------------------------------------------------------*/